NEI/INPO/EPRI Industrywide Benchmarking

Project

LP002

Human Performance Process Benchmarking Report

May 2001

NEI/INPO/EPRI Industrywide Benchmarking Project

Nuclear Energy Institute

Human Performance Process Benchmarking Report

May 2001

Acknowledgements

The Nuclear Energy Institute wishes to thank the following utilities and industry organizations for providing the personnel and resources necessary to perform this project.

American Electric Power

Arizona Public Service Company

Carolina Power & Light Company

Consolidated Edison Company of New York, Inc.

Dominion Generation

Duke Energy Generation Services

EPRI

Electricite de France

Exelon Corporation

FirstEnergy

Institute of Nuclear Power Operations

North Atlantic Energy Services Corporation

Omaha Public Power District

South Carolina Electric & Gas Company

Southern California Edison Company

Southern Nuclear Operating Company

Tennessee Valley Authority

The Detroit Edison Company

TXU

Vermont Yankee Nuclear Power Corporation

Executive Summary

Notice

Neither NEI, nor any of its employees, members, supporting organizations, contractors, or consultants make any warranty, expressed or implied, or assume any legal responsibility for the accuracy or completeness of, or assume any liability for damages resulting from any use of, any information apparatus, methods, or process disclosed in this report or that such may not infringe privately owned rights.

Benchmarking is the process of comparing one’s current practices with those of the industry leaders to achieve improvement through change. This report summarizes the results of NEI’s benchmarking effort to identify the good practices and common contributors of successful human performance improvement processes.

The resources for this project came from utility subject matter experts, Institute of Nuclear Power Operations (INPO), EPRI and NEI.  Following training in the NEI benchmarking process, the team identified industry leaders, conducted site visits, and prepared this written report.  To identify the industry leaders, a comparison of survey data collected from twenty-nine U.S. plants was balanced against the ability of the site’s to support a benchmark visit and the O&M cost. The six sites visited and the respective most outstanding human performance improvement activity were:

- Brunswick - Human Performance Continuing Training (Appendix E)

- Comanche Peak – Peer-to-Peer Observations (Appendix I)

- Farley – Weekly Human Error Investigation Process (Appendix K)

- Palo Verde - Pre-Job Briefing Database (Appendix F)

- Seabrook - Organizational Communications (Appendix S)

- Watts Bar - Excellence in Performance Program (Appendix U)

Central to human performance improvement is the “VALUE. Model” (Section 2). It is characterized by a clear Vision and Achievement of business goals and also by Leadership upholding high standards for performance and Understanding of human performance such that Event-free performance can be effectively promoted and attained.

The following common factors are effective mechanisms to improve human performance:

- Management sponsorship and leadership driven improvement initiatives

- Business planning process that integrates a human performance improvement strategy

- Communication that facilitates excellence in human performance

- Training and personal development of knowledge and skills aimed at error prevention

- Established standards and expectations for use of human performance error prevention tools

- Immediate positive reinforcement to personnel exhibiting correct behaviors

- Pre-job briefing process using data base tools and industry operating experience

- Observation programs focused on the removal of barriers to excellent performance

- Integrated self-assessment of human performance improvement activities to improve their effectiveness.

The team developed a Human Performance Improvement Process Map (Section 4) that may be used as a business tool to assess and adjust human performance improvement efforts. Additionally, good practices identified by the team are described in the appendices and annotated to show alignment with the process map.

Table Of Contents

Executive Summary	i

1	Introduction	1

1.1	Overview	1

1.2	Site Selection Process	2

1.3	Common Contributors	2

1.3.1	Management Sponsorship and Leader Driven	2

1.3.2	Integrating Human Performance in Business Planning	2

1.3.3	Communications	2

1.3.4	Training and Development	3

1.3.5	Reinforcement	3

1.3.6	Human Performance Tools	3

1.3.7	Pre-Job Briefings	3

1.3.8	Observation Programs	3

1.3.9	Integrated Self-Assessment of Human Performance	4

1.4	Plant Visit Highlights	5

1.4.1	Brunswick	5

1.4.2	Comanche Peak	6

1.4.3	Farley	8

1.4.4	Palo Verde	10

1.4.5	Seabrook	12

1.4.6	Watts Bar	13

1.5	Other Good Practices	14

2	Value Model	15

3	Common Contributors	17

3.1	Management Sponsorship and  Leader Driven	17

3.2	Integrating Human Performance in Business Planning	19

3.3	Communications	20

3.4	Training and Development	20

3.5	Reinforcement	21

3.6	Human Performance Tools	22

3.7	Pre-job Briefings	22

3.8	Observation Programs	23

3.9	Integrated Self-Assessment of Human Performance	23

4	Process Map	25

4.1	Topical Areas	25

4.2	Terminology	26

4.3	Performance Indicators	26

4.3.1	Personnel Safety (1.1, 1.2, 3.2, 3.7)	26

4.3.2	Personnel Error Rate (1.2, 3.4, 3.7, 3.8)	26

4.3.3	Significant Personnel Error Rate (1.2, 3.4, 3.7, 3.8)	26

4.3.4	Human Performance Awareness (3.1, 3.2, 3.2,  3.6, 3.7)	26

4.3.5	Backlog Management (3.1, 3.4, 3.6, 3.7)	27

4.3.6	Workplace Culture (3.1, 3.2, 3.3, 3.6, 3.7)	27

4.3.7	Learning Culture (3.6, 3.7)	27

4.3.8	Procedure Noncompliance Rate  (3.1, 3.3, 3.6, 3.7, 3.8)	27

4.3.9	Human Performance Success  (3.1, 3.2, 3.6, 3.7)	27

APPENDIces

A.  Site Selection Process	A-1

B.  Site Profile Matrix and Organization Charts	B-1

C.  Task Force List	C-1

D.  Change Management	D-1

E.  Human Performance Continuing Training	E-1

F.  Pre-job Briefing Databases	F-1

G.  Eliminating Latent Weaknesses in Jobsite Conditions	G-1

H.  Common Precursors to Errors Report	H-1

I.  Peer-to-Peer Observations	I-1

J.  Plant Event Review Committee	J-1

K.  Weekly Human Error Investigation Process	K-1

L.  Performance Indicators	L-1

M.  Management Observation Training Program	M-1

N.  Health Physics Observation Program	N-1

O.  "Focus on Five" Prevent Event Briefing Questions	O-1

P.  Hands-On Safety Behavior Laboratory	P-1

Q.  Human Performance Marketing	Q-1

R.  Quarterly Ownership and Effectiveness MeasureS	R-1

S.  Organizational Communications	S-1

T.  DeskSTAR	T-1

U.  Excellence in Performance Program	U-1

V.  STAR 7	V-1

W.  Training Alignment to Improve Human Performance	W-1

X.  Error-Likely Days	X-1

Y.  Outage Initiatives	Y-1

Z.  Glossary	Z-1

Figures

figure 2-1  value model	15

Figure 4-1  human performance improvement process map	28

figure a-1  site selection plot	a-2

Figure b-1  brunswick organization chart	b-2

Figure b-2  comanche peak organization chart	b-3

Figure b-3  farley organization chart	b-4

Figure b-4  Palo verde organization chart	b-5

Figure b-5  seabrook organization chart	b-6

Figure b-6  watts bar organization chart	b-7

Human Performance Process Benchmarking Report

# Introduction

## Overview

Between January and May 2001, a group of industry experts conducted a Human Performance Process Benchmarking Project.  The scope of the process investigated is sub-process LP-002 as described in the report – A Standard Nuclear Performance Model – The Process Management Approach, Revision 1, December 2000.

The objectives of this project were to:

- Perform a baseline evaluation of human performance.

- Identify and develop a process map.

- Select and visit at least six sites.

- Identify specific common practices and individual site good practices.

- Share process results across the nuclear industry.

This report provides the results of benchmarking visits to Brunswick, Comanche Peak, Farley, Palo Verde, Seabrook, and Watts Bar sites.  The teams conducted interviews based upon process map areas of interest.  Interviewing teams then obtained additional details to describe the practices.

The benchmarking process used the aggressive and challenging schedule to reduce the time required to achieve results.  Project task force consisted of human performance subject matter experts from 17 plants, four representatives from the Institute of Nuclear Power Operations (INPO), representatives from EPRI and NEI and site visit coordinators. (Appendix C).

Task force personnel participated in a two-day training session and a three-day scope definition meeting before conducting the site visits and the data collection.  Two-day site visits were conducted over a six-week period.  The team prepared the draft report in a final three-day review meeting.

## Site Selection Process

Sites were selected using three steps: screening, evaluating human performance benchmarking survey scores versus overall O&M cost, and finally site selection.  All domestic plants and selected international plants were invited to respond to the selection survey.  Sites failing to complete the survey or unable to host a benchmarking team were removed from consideration.  Point values were determined by subjectively scoring the surveys and normalizing the results to 100%.  Site scores were plotted against O&M costs based on EUCG O&M Cost data. and the first four  sites were selected.  Team members then nominated four additional sites and selected two sites from the additional candidates (Appendix A).

## Common Contributors

### Management Sponsorship and Leader Driven

Senior executives and managers assumed visible and dynamic sponsorship of human performance improvement.  Management established the vision and goals, and then stepped aside to let the leaders drive implementation.  Based on the management vision and goals, station leaders effectively implemented improvements that were embraced by all levels of the organization.  The management/leadership team aligned the standards, expectations, and commitment for improvement in all aspects of the organization

### Integrating Human Performance in Business Planning

The business planning process is the method that ensures the business goals become the clear direction for the organization.  Imbedding human performance into the business plan creates strategic human performance goals and identification of the major steps to accomplish these goals.  Resources are formally identified to fund the required key activities.  These key activities and processes are identified and receive required management ownership.  The business plan creates management commitment, reinforcement, and accountability, which are critical for success.  Integration of human performance demonstrated the required organizational commitment that facilitates the culture change.  At the sites visited, human performance improvement was integrated within the business plan and goals.

### Communications

Most stations visited used a structured communications approach as a means to proactively enhance overall performance at the station.  Factors relating to human performance are incorporated into daily or weekly discussions such as error precursors, error-likely situations, and available human performance tools.  Most stations used multiple media dissemination methods to ensure the messages would be viewed and used by as wide a site population as possible.  A common characteristic of successful communication includes active participation by management, supervision, and peers.  In addition, it appeared that the communication was more readily absorbed if the particular work group’s supervisor or a peer provided the message.

### Training and Development

A common characteristic among the high performing sites was a commitment towards initial and continuing human performance training.  Supervisors and managers usually presented the initial training program to all employees.  Another aspect was “Mentoring for Success” where inexperienced individuals learn from the more experienced personnel.

### Reinforcement

Most stations visited provided some type of positive reinforcement to plant personnel for exhibiting correct behaviors.  Reinforcement of observed behaviors by managers, supervisors or peers was also timely, i.e. immediate.  Reinforcement ranged from inexpensive items to certificates that could be redeemed for merchandize.  In addition, some stations conduct outage related reinforcement to enhance the performance of both plant personnel and contractors during outages.  Common characteristics of successful reinforcement systems include prompt identification and reinforcement of the desired behavior, sharing the reinforced behavior and the type of reinforcement with the rest of the site and allowing peers to either nominate or provide the reinforcement.

### Human Performance Tools

A common aspect among the benchmarked sites was the identification of a standard set of error reduction tools.  These tools were typically listed and defined within a specific procedure or guideline where expectations regarding their use were communicated.  The structure of these procedures or guidelines focused on providing a list of standard error reduction tools that can be applied to specific error precursor conditions.

### Pre-Job Briefings

Most stations visited use pre-job briefings as a means to proactively enhance overall performance at the job site.  A common characteristic of success included active participation by all attendees in discussions related to critical elements of the task, error traps, assumptions, potential consequences if not performed correctly, and contingencies to mitigate consequences.

### Observation Programs

Plant supervisory personnel routinely conducted human performance observations.  Observations focused on proper work practice behaviors as well as human behaviors that promote event free performance.  Observation trends, both on a plant and a group level, were provided to management for evaluation and action.  Observation training improves the ability of the observers to improve performance.

### Integrated Self-Assessment of Human Performance

Currently discrete, but not necessarily comprehensive methods exist for the continuous improvement of human performance through self-assessment and related processes.  The self-assessment program is a key driver for improving plant performance.  When coupled with corrective action programs, trending activities and human performance methods all work processes may be systematically optimized.  The benchmarking team recommends such a structured approach, but was unable to identify a comprehensive example during site visits.  However the team saw enough discrete examples of this activity to credit all sites collectively for establishing an integrated methodology.

## Plant Visit Highlights

### Brunswick

Brunswick was successful in developing an integrated approach to human performance improvement.  Their overall organizational and strategic planning process drove, in part, this success.  However, several other key activities that supported this approach are described below.

Human performance initiatives having widespread impact were developed using strategic business planning.  Results of periodic cultural surveys and manager input establish annual business goals.  The business plan was then communicated at a site-wide meeting.  An additional site-wide meeting, held six months later, communicated progress in achieving these goals.  The management team is aligned to human performance goals by assigning specific sponsors or leads for each focus area.  In addition to this planning and sponsorship process, the site developed a Continuous Improvement Plan (CIP) process that coordinates corrective action, operating experience, self-assessment and benchmarking program efforts in a step-by-step and integrated manner.  Benefits include schedule coordination among benchmarking, self-assessment, audit, and human performance improvement initiatives throughout the business cycle.

An extensive manual was developed to help guide individuals involved with changes using a site approved change management model. This model considers four major aspects for successful change: assessment, communication, affiliation, and capability. (Appendix D)

Human performance fundamentals’ training is central to initial and continuing improvement efforts.  Personnel receive from eight to sixteen hours of initial training with expectations of approximately eight hours of continuing training per year. Alignment is further demonstrated through the use of human performance “Workout Days”.  These special days are scheduled once per quarter and are centered on reinforcing a wide variety of human performance expectations.  Continuing training is currently integrated into the Workout Days. Appendix E).

The site has developed a highly flexible positive reinforcement process.  Budgets allocate a dollar amount per person per quarter allowing managers and supervisors discretionary flexibility to reward performance-related behaviors.  Individual performance measures typically involve gift certificates with the goal of personalizing the reward as much as possible.

A front-line employee developed database helps the organization to become fully engaged in implementing various elements of pre-job briefs and post job critiques.  Expectations for job performance require accessing the information in the database to support briefings and maintain the content with on-going in-house experience. (Appendix F)

Brunswick has expended major efforts to eliminate latent weaknesses in jobsite conditions.  For example, an integrated effort to reduce scram frequencies resulted in numerous plant modifications, development of training booklets, customer surveys and focus on the issue in Human Performance Week. (Appendix G).

### Comanche Peak

Comanche Peak has a mature human performance program with ownership down to the lowest levels of the organization and strong, consistent management commitment.  The management endorses a culture that provides the time to perform a change or to do a task right the first time.  The front line worker is quick to acknowledge that management sets the goals for human performance success, but that they are responsible to achieve it.  Over the years, there has been a strong motivation to change practices, processes and programs to improve human performance based on a few significant events that have occurred.  The staff has a strong pride in the station and its success.  Most of the station’s supervision has a clear understanding of the human performance behaviors they want practiced and they monitor and coach to anchor them.  The human performance group conducts analysis of the station event trends and provides output in various forms, which the station staff uses to enhance their ability to prevent future events.  Overall event trends have been declining since the program inception in 1992.

Management’s commitment to a team-based change management approach involving front-line ownership, has been demonstrated by the response to a crane incident last year.  Management took members of the staff out of their positions for a month to develop a new crane control program.  The front-line experts developed action plans and presented them to senior management, who wholeheartedly supported it.

Comanche Peak has a strong inclination toward a team approach to change management and work.  With the institution of cross-functional repair (“SMART:”) teams in maintenance and “Fix-it Now” (“PROMPT”) teams in engineering, the station organization is aligned to support change that results in event reduction.

Part of Comanche Peak’s success is based on the aging, long term, experienced workforce rather than pure process structure.  The management has recognized a need to pass on the knowledge of the master craftsman to the new workers for continued success and continuous improvement.  Master craftsman mentoring has been instituted in operations and engineering to accomplish this goal.  Other groups are soon to follow this practice.

The station human performance program started in 1992 with the Performance Enhancement Program as a result of a plant incident. This is made up of the following four elements:

- “All personnel at CPSES want and try to do a good, successful job”.

- Human errors cannot be eliminated, but can be managed.

- Human errors are managed by accurately identifying and correcting root causes for the human error issues.

- The severity of the event is dependent on the timing and location of task performance; the causes of significant and non-significant human error events are the same.

Common Precursors to Errors — identification and prevention, are determined by monitoring and analyzing the human performance-caused events during outages.  These lessons learned are turned into a list of common precursors that caused the station staff difficulties in past outages.  In addition to the precursors, tips on how to avoid falling into a trap for the precursor is provided for each one.  The list is published by the station root cause consulting engineer prior to the next outage.  Many departments, for example the refueling team, use the list for outage pre-job briefs and it is generally posted prominently in work areas (Appendix H).

Comanche Peak has an employee driven peer-to-peer safety observation program (BEAR).  An employee volunteering to monitor fellow employees for at-risk behavior is the essence of the program.  Employees are provided observation training prior to performing any observations.  Management observations are not a part of this process (Appendix I).

The Plant Event Review Committee (PERC) is an effective management process used to quickly perform an initial evaluation of any event involving human performance problems.  The PERC is defined in a station guideline and has been in use for over ten years.  The PERC, by definition, has four primary goals:

- Stabilize the situation.

- Communicate relevant information to concerned parties.

- Restore or repair damage.

- Define the first step in preventing future similar events (Appendix J).

### Farley

The human performance improvement program at Farley has developed over the last several years as the result of a strong, clear, simple message from executive management.  The message, “No disciplinary action for an honest mistake”, is clearly understood at all levels of the organization and has been demonstrated by the management team at the site and at corporate headquarters.  This change in management behavior has evolved to a culture of reporting low-level issues.

The simplicity of the human performance message has also carried over into the performance expectations for each Southern Company site.  Goals defined include the following:

- No effect on capacity factor from human performance deficiencies

- No effect on capacity factor from equipment performance problems

- Short refueling outages

- INPO Excellence rating

Management sponsorship of human performance has resulted in a very clear alignment of the organization concerning goals, expectations and performance in the area of human performance.  These expectations are well understood and accepted by management and the workforce.  The management team also recognized the need to change their behavior and developed a simple plan containing the following attributes:

- Establish clear expectations.

- Talk about them all the time.

- Measure how well you are doing.

- Increase your presence.

The management team also developed a simple measurement instrument providing feedback to managers concerning their accessibility, responsiveness to concerns, and quality of listening skills.

In addition to the recognition that management behaviors needed to change, Southern Company has dedicated significant resources to human performance improvement.  These include human performance champions in each department, a human performance coordinator, observation training for managers, supervisor and workers, leadership training, development of leading indicators, new employee orientation heavily weighted on human performance, and positive reinforcement programs.

Human Performance Review Boards (HPRB) are a means to thoroughly understand the human performance and organizational factors involved in events.  Management has demonstrated at these boards their commitment to principles described above. The HPRB normally meets once per week and is composed of the plant general manager, other designated managers and support personnel and the supervision and individuals involved in selected Human Performance events.  The board is based on the INPO “Excellence in Human Performance” principles.  The purpose of the board is to encourage open discussion about the event in a non-punitive environment. These boards have been effective at helping the Farley team improve plant processes and reinforce to plant personnel the emphasis on fixing the process to prevent future errors (Appendix K).

Leading Indicators have been developed with the support of EPRI to monitor those factors that potentially predict future human performance and organizational health.  Those factors thought to most affect human performance were identified and trends were developed on both a station level and a departmental level.  Lagging indicators were also selected to validate the leading indicators. As a result, work improvements have been noted in management observation quality and quantity and in types of corrective actions taken in human performance events (Appendix L).

Observation and intervention training is used at the plant to improve human performance. Plant supervisory personnel (foreman and above) conduct a minimum of one human performance observation per month.  The observation form focuses on human behaviors that promote event free performance as well as work practice behaviors.  Observation trends, both on a plant and a group level, are provided in a quarterly summary. A two-day human performance observation course was taught to all supervisory personnel and a significant number of the engineers.  This course included leadership topics, human performance fundamentals and tools, error precursors, observation process, observation techniques, confronting unwanted behaviors, practical exercises, actual observations, and sharing of lessons learned.  A one-day course was also developed from the two-day course for presentation to worker level personnel. This course has been presented to maintenance journeyman, system operators, and is scheduled for presentation to chemistry technicians, health physics technicians, the new system operators class, and to reactor operators this summer (Appendix M).

The health physics observation program was developed as a more “worker friendly” replacement for the Radiation Incident Report and is used to determine trends in radiation worker practices in order to improve those practices and to provide the training department and management with tools to help workers succeed.  Health physics technicians implement the program.  These personnel are assigned strictly to observe radiation workers for good practices as well as those that need improving.  These technicians provide on-the-spot feedback for those items performed correctly as well as coaching on practices that need improvement.  To capture observations, data from a form is collected in a computer program that outputs in both word and graphical formats.  This information is then reviewed to determine if there are trends developing within a work group or for the plant as a whole in order to stop bad habits from taking root.  A monthly report is distributed to managers and supervisors to share with their personnel (Appendix N).

### Palo Verde

Over the last four years Palo Verde has been focused on human performance improvement.  The program is named “Prevent Events”.  Focus and improvement has propagated through the active involvement of senior management.  Human performance improvement is sponsored by the Vice President, Nuclear Production, and strongly owned by the directors of each division.  Key division directors serve on their Prevent Events Steering Committee while subject matter experts from all divisions are represented on their Prevent Events Council.  These two bodies act as the change agents guiding the human performance improvement throughout the site. The most recent activities include new division specific Prevent Events Strategies, the Safety Behavior Workshop, the Operation Tailboard Database, and the “Focus on Five” Prevent Events Briefing Questions.  The tools and strategies have been effective as seen through the reduction in injuries and significant events.  Much of this success can be attributed to Palo Verde’s communications approach, management monitoring and participation, and inclusion of Prevent Events in the employee annual incentive program.

A two-tiered leadership approach to improving human performance was implemented.  A Prevent Events Steering Committee, made up of division directors, convenes as owners to establish the strategic approach for improving human performance.  This includes establishing priority of actions, monitoring key human performance-related indicators, and adjusting plans as necessary to maintain continued improvement.  A Prevent Events Council, made up of subject matter experts, convenes to facilitate division improvements, share lessons learned, and advise leaders on progress achieved.  In addition to these two groups, the site safety committee (which has existed for the life of the facility) has consistently provided strong input for improving human performance.  This group, made up mostly by workers, provides specific actions that can be implemented to improve behaviors as well as the work environment.  Palo Verde has integrated its human performance activities with its safety improvement activities, as the behaviors that support both are the same.

The current Prevent Events’ strategies reflect an evolution of the site level strategy.  Instead of one method to achieve the station Prevent Events objective, each division has developed their own vision, goals and expectations. This approach allows the divisions to identify the tools that work best for them and improves ownership of the Prevent Events Station Objective. Each division director facilitated rollout of their respective strategy and provided training on them. While the typical in-plant divisions have benefited from the personalized message from the division leaders, improved participation by the office-type workers was also noted.

“Focus on Five” is one of the strategies Palo Verde has implemented to drive home use of error reduction and event prevention techniques before performing any station tasks.  The key to this strategy is focusing the staff on the task at hand with a simple, easy-to-understand tool.  Before starting any task, from simple to complex, employees are expected to ask themselves a set of five simple questions.  By making these questions the focal point of pre-job briefs and facilitating discussion about the upcoming task, workers are better focused on the task at hand and can more readily identify issues that could cause an event.  Palo Verde senior managers and leaders play an integral part in anchoring this strategy.  They rolled out pre-job brief and two-minute drill expectations to their staff members as well as distributing lanyard-ready cards to site personnel.  Leaders follow up by observing employees’ use of the tool prior to work  (Appendix O).

In addition to the typical industrial safety classroom or computer-based training, employees can develop their knowledge and skills by performing hands-on exercises in a safety behavior laboratory.  This facility presents the safety requirements and most safety devices used in the plant.  Workers are given the opportunity to develop hazard recognition skills in a controlled, safe environment under the tutelage of a safety professional.  About fifteen focus areas are presented; some examples are ladder safety, fall protection use and electrical equipment hazards. Participation in this workshop is mandatory for all contractors before the outages and personnel from many of the in-plant divisions.  This “Safety Behavior Workshop” was opened one month before the last outage.  The number of accidents recorded dropped from eleven to three during this outage (Appendix P).

The Operations Tailboard Database is an electronic enabling tool that reinforces the critical aspects of pre-job briefs resulting in improved tailboard effectiveness.  This database provides all the information associated with a task that has been previously performed including the site lessons learned, industry operating experience from the INPO database, the personnel required, and what can be expected should the task not be conducted as required.  The success of this tool is largely based on the operators supplying the needed lessons learned information and recognizing that systematic implementation yields successful results (Appendix F).

The communications group plays an integral role in human performance improvement.  The techniques and multi-faceted approach to communicating the Prevent Events message has been effective.  The use of videos, posters, newsletters, cartoons, etc., continues to engage personnel at all levels.  A recent station human performance self-assessment showed excellent employee understanding of the Prevent Events effort as well as how tools should be used (Appendix Q).

### Seabrook

Seabrook has effectively taken steps to improve human performance through a formal change management plan during the past two years.  Improvement was manifested by improved plant performance and positive personal relationships observed during the visit.  The goal of this plan was to improve the station human performance, increase ownership and understanding of human performance principles and practical applications throughout the organization and make Seabrook Station a highly productive and event-free organization.  Included in the human performance improvement process are:

- Human Performance Observation and Coaching programs

- Condition Report Coding for Human Performance

- Human Error Reduction Tools

- Human Performance Metrics

- Human Performance Review Board

- Human Performance Review Committees

- Periodic Human Performance metric reports

Management support is exhibited throughout the organization demonstrating both vertical and horizontal alignment of the Seabrook team and work processes.  Seabrook has also developed “Human Performance Improvement Process” guidelines that describe organizational strategies, policies, processes, and practices used to achieve and support excellence in human performance.

Daily communications practices are effective in establishing and maintaining organizational alignment on station priorities as well as distributing information to all employees (Appendix S).

Seabrook has a structured approach for measuring the health of the individual department’s human performance improvement efforts.  The respective department manager, aided by the department human performance coordinator, submits a quarterly “Ownership and Effectiveness Measurement” report to the station human performance manager. This measure promotes an added level of accountability for department managers and is reviewed by the site Human Performance Review Board to identify station-wide trends and areas for improvement (Appendix R).

Seabrook has created a software tool, deskSTAR, which allows individuals to practice self-checking, peer-checking, procedure place-keeping, and Stop When Unsure.  This tool was used prior to the last refueling outage to raise the awareness of plant personnel of the tools available to help reduce human performance issues (Appendix T).

### Watts Bar

Watts Bar has a very receptive and approachable organization with human performance ownership at the highest levels.  The management team is in vertical alignment with the standards, expectations, and commitment for improvement in all aspects of the organization, including a high focus on human performance improvement.  Over the years, the organization has conducted culture surveys to gain an understanding of the commitment for change and heighten awareness of the management team with regards to additional human performance improvements.  The results of the culture surveys are utilized by the management team to evaluate and modify the human performance improvement plan and maintain the positive culture.

Management has developed a strong program that identifies, recognizes, and reinforces the behaviors of the workforce in a positive and constructive manner.  The management team encourages positive behaviors with constant reinforcement to all levels of the organization.  Management has set the standard for human performance improvement to be engrained into daily activities with reinforcement of the expectations at meetings, mid-shift briefings, and sharing lessons learned with other TVA stations.  The station has developed a “Blue Card” reward program (Section 3.5).

Watts Bar has developed a structured approach to cultivate personnel understanding of and performance to expectations for operational excellence with the “Excellence in Performance Program” (EIP).  EIP provides detailed performance expectations, a method for individuals to evaluate their knowledge, a means to measure and provide feedback, with an emphasis on coaching for day-to-day conduct of key processes, including the focus on human performance. (Appendix U).

Watts Bar has adopted the STAR 7 program, which is an opportunity for team building among co-workers, their supervisors, and senior leaders.  It provides various interpersonal skills to change thinking and transform the business by describing how they treat each other and how to approach work.  The program establishes a behavioral framework for implementing TVA “Seven Values;” the core values that include integrity, respect for the individual, accountability, teamwork, innovation and continuous improvement, honest communication, and flexibility.  STAR 7 focuses on winning behaviors and performance.  This program was rolled out to the entire company, with reinforcement to the various levels of the management team.  The attributes of STAR 7 are incorporated into the manager and supervisor yearly appraisal system, along with peer feedback and manager coaching on areas for improvement (Appendix V).

Tools for human performance excellence are reinforced by line management in the training environment.  Line managers teach in various programs including human performance, leadership behaviors and soft skills like conflict management.  Operations training has developed a method to assess the needs of each operating crew prior to their training week.  Prior to the first week of each new training cycle, a “pilot” class is conducted with the operations and training management team assessing the material to ensure the proper focus is given to each operating crew.  A training instructor is assigned to each crew and begins the training week with an observation of crew performance in order to ensure proper focus are included in the operating crew training cycle (Appendix W).

## Other Good Practices

This project evaluated additional human performance practices outside those observed at visited sites.  These additional practices were described by a sponsoring team member and included in the report if it was concluded that a good practice existed.  Additional good practices in this category are the following:

- Error Likely Days – Davis Besse (Appendix X)

- Outage Initiatives – LaSalle (Appendix Y).

# Value Model

The data collected during the site visits revealed several common factors when analyzed as a whole.  In Figure 2-1, the VALUE Model illustrates the optimum balance sought by achieving high levels of performance while being mindful of overall cost.  The critical success factors identified to help achieve the balance are Vision, Achievement, Leadership, Understanding and Event free and it described more fully below.

Figure 2-1	Human Performance VALUE Model

- Vision – A clear understanding of what is needed to prevent events triggered by human error, while accomplishing production goals, is common among personnel.  This vision helps align the values, beliefs, and behaviors throughout the organization and is consistent with the station’s mission.  Station processes, management and leadership practices, and expectations are aligned with this clear vision of event-free performance.

- Achievement –The attainment of a performance level based on business results derived from desired behaviors.  Production is a key focus for the station and long-term economic survival depends on the productivity of the physical plant.  Production capability may be maximized through consistent training, planning, corrective action, self-assessment, and trending processes.  These processes identify and correct performance deficiencies.  Also, defense-in-depth is built into station processes, training, and plant equipment design to minimize challenges to the safe and reliable production of electricity.

- Leadership - Committed leadership actively promotes human performance improvement efforts.  The leaders facilitate understanding and alignment within the organization of the business plans, processes, and behaviors necessary for long-term success.  Leaders actively observe and reinforce behaviors to promote excellence in human performance.  Effective leadership results in shared values and beliefs supportive of the station’s vision.

- Understanding – The ability to discern human nature and, by this knowledge, influence human performance.  The nuclear industry’s human performance fundamentals supply a practical means to promote both error-free and event-free performance.  Error-prevention techniques are more effective when plant staff understand how their behaviors lead to the prevention of events.

- Event Free - An underlying driver for human performance improvement is to reduce the frequency and severity of significant events.  While human error cannot be eliminated entirely, the ability to operate event free is possible by establishing defense-in-depth.  Defense-in-depth is established at an organizational level, through an integrated approach to managing human performance, which aims to minimize and accommodate the occurrence of human error.  The long-term benefit to the plant is safe, reliable, and economic performance.

# Common Contributors

## Management Sponsorship and  Leader Driven

Senior executives and managers assumed visible and dynamic sponsorship of human performance improvement.  Management established the vision and goals for human performance improvement.  Senior executives and managers effectively communicated their human performance improvement message to all levels of the organization, including the front line workers.  Beyond mere communication was also the demonstrated belief by the management team of the importance of having the message reinforced frequently.  Management ownership was a critical support function to the leaders that interfaced daily with station personnel.

Based on the management vision and goals, station leaders effectively implemented improvements that were embraced by all levels of the organization.  The management/leadership team aligned the standards, expectations, and commitment for improvement in all aspects of the organization, including a high focus on human performance improvement.  This alignment, both vertical and horizontal, was one key element to the success of human performance improvement activities.

Most of the stations visited publish performance expectations in pocket size reference booklets.  These booklets were one way the leadership team enforces and aligns the organization to the organization expectations.  These booklets were typically department specific, reflecting their type of work with human performance expectations included.  To encourage employee use of the booklet, practical day-to-day useful information was included in the booklet.

Specific examples include:

- Brunswick has been successful at developing an integrated approach to human performance improvement.  Their overall organizational or strategic planning process has driven this success, in part.  The management team has effectively aligned themselves around the human performance initiatives/goals by assigning specific management sponsors or leads, for each identified focus area or improvement item. Focus areas include the involvement of department personnel in the identification of performance issues, the conduct of training, and the communication of expectations.  This coupled with an accountability process that involves department managers and supervisors in accomplishing specific focus area activity, are fundamental aspects to Brunswick’s success in managing human performance improvement.

- At Comanche Peak, management’s commitment to a team change approach, with low-level ownership, has been demonstrated by the response to a crane incident last year.  Management reassigned line responsibility for a group of front-line personnel for a month so their group could develop a new crane control program. The front-line experts developed a program, presented it to senior management and helped lead implementation.

- Expectations at Farley are well understood and accepted by management and the workforce.  The management team also recognized the need to change their behavior and developed a simple plan containing the following attributes:

- Establish clear expectations

- Talk about them all the time

- Measure how well you are doing

- Increase your presence.

The management team also developed a simple measurement instrument providing feedback to managers concerning their accessibility, responsiveness to concerns, and quality of listening skills.

- The Seabrook station leadership team has created a positive environment for continuing improvement by demonstrating what excellent plant performance looks like and engaging the workforce in achieving it.  This effort provides personal development training for all employees and actively involves individuals at all levels of the organization in making improvements.  As a result, initiatives in many areas are more complete and better accepted by the workforce.  One example includes the development of written guidelines for improving human performance.  The guidelines include organizational strategies, policies, processes and practices used to achieve and support excellence in human performance.  The guidelines identify the roles and responsibilities for each employee (manager, supervisor, and individual) to improve performance.

- The Watts Bar plant has a very receptive and approachable organization with human performance ownership across the organization.  The management team has gained vertical alignment through the establishment of standards, expectations, and commitment for all aspects of organizational improvement.  Over the years, the organization has conducted culture surveys to gain an understanding of the commitment for change and to heighten awareness of the management team with regards to additional human performance improvement activities.  The management team evaluates and modifies the human performance improvement plans in order to maintain a positive culture.

## Integrating Human Performance in Business Planning

The business planning process ensures the business goals become clear direction for the organization.  Imbedding human performance into the business plan creates strategic human performance goals and identification of the major steps to accomplish these goals.  Resources are formally identified to fund the required key activities.  These key activities and processes are identified and receive management ownership.  The business plan creates management commitment, reinforcement and accountability that are critical for success.  Integration of human performance activities demonstrates organizational commitment that facilitates culture change.  If human performance is not integrated within the business plan it can easily become a “program of the month”.  At the sites visited, human performance improvement was integrated within the business plan and goals.

Examples include:

- The business planning process at Brunswick encompasses four strategic plan initiatives.  The Human Resource Initiative (1 of 4) has been further subdivided into various focus areas with Human Performance Excellence being one such area.  In each focus area, a manager-level sponsor is identified and tasked with developing a strategy for addressing human performance excellence.

- Several inputs are used to help guide strategy development including off-site planning sessions, assessments, evaluation results and periodic employee cultural surveys.  This input is used to help identify individual, leadership and organizational level issues.  A discrete set of activities to ensure continuous progress was developed as appropriate.

- The sponsor develops a strategic plan for their area of responsibility.  This includes a set of specific activities, basic assumptions, risks, and performance measurement tools used to assess success in accomplishing the overall goal for the area of concern.

- Apart from human performance excellence, the Human Resource Initiative also addresses Supervisory Effectiveness and Leadership Skills areas for the upcoming year.

## Communications

Most stations use a structured communications approach as a means to proactively enhance overall performance at the station.  Factors relating to human performance were incorporated into daily or weekly discussions such as error precursors, error-likely situations, and available human performance tools.  Most stations used multiple media dissemination methods to ensure the messages would be viewed and used by as wide a site population as possible.  A common characteristic of successful communications includes active participation by management, supervisors and peers.  In addition, it appeared that the communications was more readily absorbed if the particular work group’s supervisor or a peer provided the message.

Proactive outage human performance stand-down days and routine human performance awareness days are used to proactively enhance awareness of the need to reduce events and to promote various tools to prevent errors.  One station held human performance days specific to their outage activities.  An awareness day is held one week prior to the outage, with another awareness day midway through the outage to focus on human performance.

Other examples include:

- Seabrook (dissemination of daily meeting information before the craft was dispatched to the field, plant manager daily phone call, weekly human performance newsletter, and lanyard cards)

- Palo Verde (video clips, posters, messages on closed circuit TV, and lanyard cards),

- Comanche Peak (pre-outage “Common Precursors to Errors”).

## Training and Development

A common theme among high performing utilities is a commitment towards initial and continuing human performance training.  Supervisors and managers usually presented the initial training program to all employees.  The INPO Fundamentals for Human Performance training course provided the framework for this training and other material was added to make the training applicable to day-to-day activities.  This was accomplished by providing specific examples of error precursors as well as the tools available to prevent errors that were applicable to the work group receiving the training.  Human performance training is also provided on a continual basis.  This type of training takes place in many forms including classroom topics, “workout days” and supervisory training.

Another important aspect was the development of the workforce.  Since the nuclear industry represents a maturing work force, “Mentoring for Success” has become a method to develop individuals through learning from more experienced employees.  One technique used by the station to perpetuate continuous improvement was mentoring by master workers.  These masters pass on the embedded “tribal knowledge” to the newer workforce members.

Additional examples include:

- At Farley a two-day human performance observation course was taught to all supervisory personnel and a significant number of the engineers.  The course included leadership topics, human performance fundamentals and tools, error precursors, observation process, observation techniques, confronting unwanted behaviors, practical exercises, actual observations, and sharing of lessons learned.  Additionally, a one-day course structured after the two-day course was presented to all other individuals.

- Watts Bar tools used for human performance are reinforced in the training environment, with line management ownership demonstrated.  The line managers provide training in various programs including human performance, leadership behaviors, and soft skills like conflict management.  In addition, the operator training process has developed a method to assess the needs of each operating crew prior to their training week.

## Reinforcement

Most stations provided some type of positive reinforcement to plant personnel for exhibiting correct behaviors.  Reinforcement of observed behaviors by managers, supervisors or peers was also timely, i.e. immediate.  Reinforcement ranged from small gifts to certificates that could be redeemed for merchandise.  In addition, some stations conduct outage-related reinforcement to enhance the performance of both plant personnel and contractors.  Common characteristics of successful reinforcement systems include prompt identification and reinforcement of the desired behavior, sharing the reinforced behavior and the type of reinforcement with everyone and allowing peers to either nominate or provide the reinforcement.

Noteworthy practices in this area include:

- Good Catch Awards at Comanche Peak and Seabrook,

- Prevent Event Awards and inclusion in annual bonus programs at Palo Verde

- Blue Card certificates of accomplishment and recognition along with a gift card to redeem station items at Watts Bar

- Outage incentives at LaSalle

- Quarterly human performance drawings and green card awards at Davis Besse.

## Human Performance Tools

Another common aspect was the identification of a standard set of error reduction tools.  These tools were typically listed and defined within a specific procedure or guideline where expectations regarding their use were communicated.  The structure of these procedures or guidelines focused on providing a list of standard error reduction tools that can be applied to specific error precursor conditions.  The extent and specific description for each selected error reduction tool may vary from site to site, however certain common tools were noted as follows:

- Task Preview (similar to INPO’s SAFE method)

- Self-Checking

- Peer Checking

- Two and Three-Way Communication

- Verbalization

- Stop and Collaborate

- Devils Advocate/Challenge

- Change Dialogue

These types of tools were typically promoted through training and other communication methods to reinforce specific performance expectations.  The tools were supported by strong management reinforcement and worker recognition of the observable benefit of their use.

## Pre-job Briefings

Most stations used pre-job briefings as a means to proactively enhance overall performance at the job site.  The briefings provide an opportunity to anticipate error-likely situations, reinforce expectations, review appropriate operating experience, and review the error prevention tools as they apply directly to the task to be performed.  A common characteristic of success included active participation by all attendees in discussions related to critical elements of the task, error traps, assumptions, potential consequences if not performed correctly and contingencies to mitigate consequences.

Stations visited that exhibited noteworthy practices in this area include:

- Seabrook (four questions for all tasks)

- Palo Verde (Focus on Five and Two Minute Drill)

- Brunswick and Palo Verde (comprehensive pre-job briefing databases).

## Observation Programs

Plant supervisory personnel routinely conducted human performance observations.  Observations focused on proper work practice behaviors as well as human behaviors that promote event free performance.  Observation trends, both on a plant and a group level, were provided to management for evaluation and action.  Observation training improves the ability of the observers to improve performance.

Individual performance of observations is tracked to ensure observations are performed.  The observation results for the plant and each group are trended and the strengths, areas for improvement and significant comments and recommendations are summarized.  Action items are tracked to closure.  Department heads analyze their data and identify appropriate actions for department improvement.

Observation training courses provide managers and supervisors the tools necessary to perform effective observations.  Additionally, plant personnel are provided information concerning the observation process.  This includes the expectations of managers toward performing observations.  To ensure quality, management monitors the conduct of observations and also the results.  Plant managers also periodically monitor the observers to ensure high standards are maintained.

## Integrated Self-Assessment of Human Performance

Methods exist for the continuous improvement of site human performance through self-evaluation.  The self-assessment program is a key driver for improving plant performance, processes and human performance.  A structured process is used for self-assessment activities.  This same process should be used to assess aggregate station human performance. Inputs to the human performance self-assessment activities include:

- Other plant self-assessment activities/results

- Corrective action performance and trend data

- Operating experience data

- Benchmarking activities/results

- Observation data.

Results from the assessment of information can be used to identify the underlying organizational issues that challenge excellent human performance.  Actions to address these issues are input elements to the business planning process.  While all stations visited used human performance trend data to identify organizational issues as well as portions of others methods discussed above, a comprehensive application of all techniques was not yet in place at any site.  However the team saw enough examples of this activity to credit all sites collectively for establishing this integrated methodology.

.

# Process Map

A process is the set of actions necessary for converting resource inputs into desired outputs and the actions are performed by someone or something under defined conditions.  A process map is a tool to describe the scope, core activities, conditions, and relationships of the particular process.  The process map below illustrates the organizational nature of human performance.  When human performance is perceived as a process, improvement in human performance is more methodical and lasting.

The benchmarking team developed the Human Performance Continuous Improvement Process Map by identifying and grouping related activities identified by The Standard Nuclear Performance Model – A Process Management Approach, Revision 1, December 2000.  The map, Figure 4-1, provides a concise overall reference for sub-process LP-002 as it relates to human performance.  Questions used by the site visit teams were derived from respective core activities on the process map.  Good practices are cross-referenced to the process map.

## Topical Areas

The map contains four overall process categories to meet the business need:

- Program Administration (1.0):

- Station mission and vision (1.1)

- Business planning (1.2)

- Change management (1.3)

- Program Guidance (2.0):

- Regulatory requirements (2.1)

- Industry guidance (2.2)

- Other guidance (non-regulatory) (2.3)

- Core Activities (3.0):

- Expectations (3.1)

- Management practices (3.2)

- Leadership practices (3.3)

- Station processes (3.4)

- Task-specific human performance process (3.5)

- Performance monitoring (3.6)

- Identify performance gaps (3.7)

- Causal assessment (3.8)

- Adjust plan (3.9)

- Program Evaluation (4.0):

- Self-assessment (4.1)

- Benchmarking (4.2)

- External oversight (4.3)

## Terminology

A glossary of terms is provided in Appendix Z.

## Performance Indicators

The team collected key performance indicators during the benchmarking visits. The sites identified and trended both a station level and departmental level those factors thought to most affect human performance.  Performance indicators varied from site to site, and as expected, varied based upon the management team’s needs, expectations, and where management wanted to focus attention.  The performance indicators referred to below are cross-referenced to the process map (map numbers shown in parenthesis).

### 4.3.1	Personnel Safety	(1.1, 1.2, 3.2, 3.7)

Most stations measured industrial safety accident rate and OSHA recordable injury rate.  Some also track employee hours without a lost time injury.

### 4.3.2	Personnel Error Rate	(1.2, 3.4, 3.7, 3.8)

Total personnel error rates on a rolling one-year average are being collected.  Most stations base this on errors per 10,000 hours to normalize the increased man-hours worked during outages.

### 4.3.3	Significant Personnel Error Rate	(1.2, 3.4, 3.7, 3.8)

Total significant personnel error rates on a rolling one-year average are being collected.  Most station based this on errors per 10,000 hours to normalize the increased man-hours worked during outages.

### Human Performance Awareness	(3.1, 3.2, 3.2,  3.6, 3.7)

The following human performance awareness measures were identified:

- Total hours of human performance training received per employee during a specific period of time, usually one month.

- Total number of human performance observations that were done by management/supervision during a specific period of time, usually one month.

- Total hours of overtime/excess straight time per employee by department.

- Total number of executive visits to plant during a specific period of time, usually one month.

### Backlog Management	(3.1, 3.4, 3.6, 3.7)

Performance indicators related to management of backlog were as follows:

- Total number of procedure revision requests received during a specific period of time, usually one month.

- Total number of component label requests made during a specific period of time, usually one month.

- Total number of open corrective actions during a specific period of time, usually one month.

### 4.3.6	Workplace Culture	(3.1, 3.2, 3.3, 3.6, 3.7)

Performance indicators related to workplace culture were as follows:

- Percentage of corrective actions that were “coach the individual” (negative reinforcement) verses “correct the process”.

- Percentage of corrective actions that were self-identified verses those identified by others.

- Percentage of planned to unplanned work (emergent work)

### Learning Culture	(3.6, 3.7)

Total number of self-assessments, benchmarking trips and assist trips completed by department during a specific period of time.

### Procedure Noncompliance Rate 	(3.1, 3.3, 3.6, 3.7, 3.8)

This indicator depicts the procedure noncompliance rate per 1,000 man-hours, as identified by the corrective action program.

### Human Performance Success 	(3.1, 3.2, 3.6, 3.7)

Success of human performance activities was monitored as follows:

- Human Performance Success Rate is the number of significant personnel related incidents per 1000 man-hours

- Human success leading indicator rate is a three-month rolling average of personnel related corrective action reports.  This indicates that non-significant deficiencies are being identified and documented at a level where precursors to declining/adverse performance can be identified.

Figure4-1 Page 1	Human Performance Improvement Process Map - Overview

Figure 4-1 Page 2	Program Management

Figure 4-1 Page 3	Program Guidance

Figure 4-1 Page 4	Core Activities

Figure 4-1 Page 5	Program Evaluation

Figure 4-1 Page 6	Step 3.5  Task-Specific Human Performance process Map

Figure 4-1 Page 7	Steps 3.7 - 3.9  Identify and Evaluate Performance Gaps

APPENDIX A

Site Selection Process

The team developed selection criteria to identify at least six good performing plants in diverse geographic locations in the United States.  Additionally, the task force considered nuclear units outside the United States and industries other than nuclear power generation.  The report was prepared and based on data and observations gathered during site visits.  This appendix describes the selection process.

The team developed a site selection survey, consisting of thirty-five questions.  This survey was designed for electronic responses via NEI’s Web page and consisted of short answer and fill-in-the-blank questions that could be completed in about two hours.  Each U.S. nuclear plant and selected plants outside the U.S. were requested to respond to the survey within two weeks.  In addition to the survey, the respondents were asked to select site visit windows from a schedule included in the survey.  Twenty-nine U.S. plants and three plants outside the U.S. responded, although some respondents were unable to support the visit schedule provided.  The survey questions appear at the end of this appendix.

At a second meeting, the team reviewed the survey responses.  The team determined which questions in the survey would be scored and assigned point values for each of those questions.  Additionally, the questions were grouped into common topics and the common topics were weighted based on the importance of that topic to the human performance program.  The team then scored each individual question response subjectively from zero to full credit.  To ensure consistency in scoring, the questions were divided into groups and sub-teams were assigned to score each group of questions for each plant. The raw scores were modified using the weighting previously noted and then normalized to 100%. A survey score was calculated for each plant.  The maximum possible survey score was 100 points.

The team noted during the scoring that some of survey responses for plants outside the United States were incomplete or unexpected (i.e. did not answer the question, missed the intent of the question, etc).  After discussion within the team, particularly among members with international experience, the team determined that the survey was written in a way that made it easy for U.S. plants to respond but difficult for those plants outside the U.S.  The team concluded that the survey would not be an appropriate tool for determining site selection for plants outside the U.S.  An alternative selection process considering plant performance and expert opinion would be used.

To aid in selecting plants for site visits, the survey score was plotted against O&M cost. (Figure A-1).  Quadrant lines represent median values.  Plants where O&M data was not available were plotted conservatively.  The team reduced the desired quadrant to select four plants.  Team members than nominated four additional plants based on the profile data supplied in the survey and other factors such as INPO identified strengths and recent experiences of the team members.  The team then voted for two of the four, resulting in a total of six selected sites.

The following six plants were selected for visits: Brunswick, Comanche Peak, Farley, Palo Verde, Seabrook and Watts Bar.

Figure A-1 - Site Selection Plot

Horizontal Axis- Human Performance Survey Points

Vertical Axis- O&M Cost in cents per kilowatt-hour (cost increasing down)

Human Performance Benchmark Survey

Please identify a contact person for follow-up information concerning survey data:

Name _____________________

Plant (s)___________________

Telephone __________________

Email _____________________

Fax _______________________

If you are interested in being visited by the team, please complete the following section. In order to compress the time taken to schedule plant visits please tell us now which one or more of the following date windows are acceptable. During the visit, a 30 minute to one-hour interview will be requested for a sampling of plant personnel.

Week of February 26, 2001  Y/N____

Week of March 5, 2001 Y/N____

Week of March 12, 2001  Y/N____

Week of March 19, 2001 Y/N____

All interviews will be requested between 7:00 a.m. and 5:00 p.m.

If your site is selected for a visit, the team would appreciate a dedicated point of contact for coordination of interviews and other logistical matters. We recommend the Human Performance Coordinator or Lead Human Performance Line Manager for this interface. We will develop a Site Visit Plan for each selected site by February 16, 2001.

Program Administration

- [0-5 points based on formality of program] How are your human performance improvement activities defined?

- Formal program description

- Informal set of processes

- Other ______________________(describe)

- [0-5 points based on age of program] If your human performance program is formally defined, how long has it been in place?

- 0-6 Months

- 7-18 Months

- Greater than 18 months

- [0-5 points based on level of management] Who is the Sponsor of human performance improvement.

- Corporate Executive

- Site VP

- Other site executive

- Plant Manager

- Plant manager direct report

- Human Performance Coordinator/Manager

- Other __________________________Describe)

- [Profile 0 points] Describe how your human performance improvement activities are governed.

- Human performance steering committee,

- Senior leadership team,

- Line department owned,

- Corporate steering committee,

- Grass roots leadership,

- Other  ________________________________

- [ Profile 0 points] For what primary purpose is your top level management interested in human performance?

Correct known performance issues

Sustain high performance

Improve productivity/efficiency

- [0-5 points] Please list business plan or strategic elements directly related to human performance improvement.  List up to five items, in order of importance/perceived benefit.   (leave blank if none)

- ________________

- ________________

- ________________

- [ Profile 0 Points] Please provide the following information:

- Number of employees plus contractors at site (non outage)  _____________(number)

- Number of full-time equivalent employees dedicated to human performance improvement activities. ________________(Number)

- Budget for positive reinforcement programs.   ______________(dollars)

- [0-5 points] What do you consider to be the primary strength(s) of your human performance efforts?  .

- List up to five items, in order of importance/perceived benefit.

- ________________

- ________________

- ________________

- ________________

- ________________

- [Profile 0 points] What do you consider to be your greatest opportunities for human performance improvement?  List up to five items, in order of importance/perceived benefit.

- ________________

- ________________

- ________________

- ________________

- ________________

- [ 0-5 points based on use and actions] Do you use culture surveys?      Y/N

If yes, how frequently are they performed?

(0-1  year)

(2-3) years)

(4 or more)

11.[ scored as part of question 10] If yes,  Did you act on the culture survey results?    Y/N

If yes,  describe the actions taken with the most recent culture survey results?

12.[ 0-5 points] Describe strengths in managing the work environment that has provided improved performance.

(i.e. labeling, procedures, workarounds reductions, housekeeping, equipment access, ergonomics, etc.):

13.[0-5 points] Have you simplified station processes that have resulted in improved human performance within the last 3 years?      Y/N

If yes, describe?

14.[0-5 points based on number used] What tools / processes do you use to help manage Human Performance activities.  (check all that apply)

Self assessment

Human Performance

Observation

Corrective action program

Self checking

Peer checking

Pre-job briefing

Post Job Briefing

Problem solving

Communication

Teamwork

Decision Making

Meetings

Change Management

Other…(please list)

15.[0-5 points] Of those indicated above which have been most useful and why?  (May list more than one)

16.[0-5 points based on percentage] Do you provide training on Human Performance and why people make errors (e.g.  INPO/WANO Human Performance Fundamentals).    Y/N

If yes,  What percentage (to nearest 25%) of your site has received this training?

25%        50%           75%          100%

17.[0-5 points based on hours] If yes, please indicate which of the following groups received the initial training and how many hours is the class:

Initial training

Executives      0   < 2 hrs   2-4 hrs   4-8 hrs   8-16 hrs    >16 hrs

Managers        0   < 2 hrs   2-4 hrs   4-8 hrs   8-16 hrs    >16 hrs

Supervisors    0    < 2 hrs   2-4 hrs   4-8 hrs   8-16 hrs    >16 hrs

Individuals     0     < 2 hrs   2-4 hrs   4-8 hrs   8-16 hrs    >16 hrs

18.[ scored as part of question 17] Continuing or refresher Training

Executives      0    < 2 hrs   2-4 hrs   4-8 hrs   8-16 hrs    >16 hrs

Managers        0    < 2 hrs   2-4 hrs   4-8 hrs   8-16 hrs    >16 hrs

Supervisors    0     < 2 hrs   2-4 hrs   4-8 hrs   8-16 hrs    >16 hrs

Individuals     0     < 2 hrs   2-4 hrs   4-8 hrs   8-16 hrs    >16 hrs

19.[Profile 0 points] Who provides human performance training?

In-house                   INPO                   Contractor

20.[ 0-5 points] How do you measure human performance training effectiveness?

_______________________________________________________________

21. [Profile 0 points] List topics that are included in the Human Performance Training…(e.g. Error Prevention, Observations, JIT, Other)

22.[ 0-5 points] Which communication mediums do you currently use to address human performance topics? (select all that apply) and are these communications usually proactive or reactive?

Proactive       Reactive

Daily tidbits					    _____            _____

Weekly Manager Notes			    _____            _____

Monthly publications		 	  	 _____            _____

HP Stand downs/timeouts			    _____            _____

Other:   _____________ 			     _____            _____

23.[ 0-5 points] How many human performance stand downs/timeouts/awareness days.

How many did you have in 2000?			_____

How many do you have planned for 2001?		_____

24.[ 0-5 points] Your corrective action process works well at identifying HP issues?

Strongly agree        agree       neutral   disagree           Strongly disagree

25.[ 0-5 points] Your Corrective action program works well at tracking and trending HP issues?

Strongly agree     Agree   Neutral    Disagree       Strongly disagree

26.[ 0-5 points] Your Corrective Action Process works well at implementing HP issue recommendations?

Strongly agree     Agree   Neutral   Disagree         Strongly disagree

27.[Profile 0 points] Select the 3 most often used corrective actions methods applied to address HP issues identified through the corrective action process.

Training

Procedure revision

Coaching

Counseling

Lessons learned

Process improvement

Plant modification

Other  _______________

28.[ 0-5 points based on examples] Is your corporate nuclear executive management team involved in human performance improvement?   Y    N

If yes,  Please list 3 specific examples that illustrate engagement.

29.[ 0-5 points based on examples] Is your site senior management team involved in HP improvement? Y/N

If yes,  Please list 3 specific examples that illustrate engagement…

30.[ 0-5 points based on indicators] Do you have Human Performance Program performance indictors? Y/N

List your program indicators and indicate if they are leading or lagging indicators

Leading	     Lagging

1. ______________________	O		O

2. ______________________	O		O

3. ______________________	O		O

4. ______________________	O		O

5. ______________________	O		O

31.[ 0-5 points based on example] What program indicator is most influential in improving human performance?

- ___________________________________

32.[ 0-5 points based on example] Have you performed a focused self-assessment at the site level on your human performance program within the last two years?Y/N

If yes , name the best improvement item….  _________

33.[ 0-5 points based on implemented changes] Have you conducted any benchmarking activities to specifically focused on human performance?Y/N

What facilities did you benchmark

___________________________________

___________________________________

___________________________________

What changes did you implement as a result of this benchmarking?

34.[5 points if yes] Has any other site benchmarked you human performance program. Y/N___

35.[ 0-5 points based on performance] In the last 3 years how many of the following have occurred as a result of human error:

- 2000           1999         1998

Reactor Scram			_______________________

Lost Generation		_______________________

Extended Outage		_______________________

LERs				_______________________

Personnel Injury/LTA		_______________________

Radiological Exposure	_______________________

36.[ 0-5 points based on examples] List and describe any human performance strengths you believe the benchmarking team should investigate (up to three topics)

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

APPENDIX B

Site Profile Matrix and Organization Charts

|  | Brunswick | Comanche Peak | Farley | Palo Verde | Seabrook | Watts Bar |
| --- | --- | --- | --- | --- | --- | --- |
| Utility | Progress Energy | TXU | SNC | APS | NAESCO | TVA |
| Multiple Sites? | Yes | No | Yes | No | No | YES |
| Operating Units | 2 | 2 | 2 | 3 | 1 | 1 |
| Staffing Level | 765 | 1,319 | 850 | 2,099 | 684+208 | 661 |
| Human Performance Program Administrator | Operations Manager | Nuclear Oversight | Licensing | Nuclear Assurance | Mgr. Performance Improvement | Plant Manager |
| Level Of Program Sponsor | Corporate Executive | V/P Nuclear Operations | VP Nuclear Operations | V/P Nuclear Production | VP | VP/PM |
| Years That The Program Has Been In Place | >18 months | 9 | 3 | 4 | 1 | <2 |
| Full Time Equivalent HU Personnel | 4 | 3 | 3 | 4 | 4 | 1 |
| Man-Hours Spent On Fundamentals | 12,240 | 11,600 | > 10,000 | 10,000 | 6,900 | 3,200 |
| HU Events In Last 3 Years |  |  |  |  |  |  |
| Plant Trips | 2 | 0 | 2 | 2 | 0 | 0 |
| Lost Generation | 1 | 12 | 2 | 0 | 0 | 2 |
| Extended Outage | 0 | 12 | 0 | 1 | 0 | 0 |
| Unplanned Radiation Exposure | 0 | 0 | 0 | 2 | 0 | 0 |

Figure B-2	Comanche Peak Organization Chart

Figure B-3	Farley Organization Chart

Figure B-4	Palo Verde Organization Chart

Figure B-5	Seabrook Organization Chart

Figure B-6	Watts Bar Organization Chart

APPENDIX C

Task Force List

| Mr. Clifford A. Berry Human Performance Program Manager North Atlantic Energy Services Company Seabrook Station P. O. Box 300 Seabrook, NH 03874 Phone: (603) 773-7662 Fax: (603) 773-7222 E-mail: berryca@naesco.com | Mr. Dan Bigsby Programs and Assessments Southern California Edison Company San Onofre Nuclear Generating Station P.O. Box 128 (D4B) San Clemente, CA 92674 Phone: (949) 368-6539 Fax: (949) 368-5195 E-mail: bigsbyda@songs.sce.com |
| --- | --- |
| Mr. Don L. Calsyn Technical Support Manager Vermont Yankee Nuclear Power Station P.O. Box 157 Governor Hunt Road Vernon, VT 05354-0157 Phone:(802) 451-3120 Fax:(802) 451-3130 E-mail: don.calsyn@vynpc.com | Mr. Tommie E. Carroll Site Root Cause Coordinator Duke Power Company Oconee Nuclear Station 7800 Rochester Highway Seneca, SC 29672 Phone: (864) 885-3200 Fax: (864) 885-3701 E-mail: tecarrol@duke-energy.com |
| Mr. Tom Chaffee Human Performance Coordinator Dominion Generation North Anna Power Station P.O. Box 402 Mineral, VA 23117 Phone: (540) 894-2843 Fax: (540) 894-2841 E-Mail: Thomas_Chaffee@dom.com | Mr. G. Phillip Crone Licensing Supervisor Southern Nuclear Operating Company Farley Nuclear Plant P. O. Drawer 470 Ashford, AL 36312 Phone: (334) 814-4712 Fax: (334) 814-4649 E-mail: gpcrone@southernco.com |
| Mr. Robert Coovert Exelon Nuclear Human Performance Director Exelon Corporation ETW III, Suite 900 1400 Opus Place Downers Grove. IL 60515 Phone: (630) 663-7629 Fax: (630) 663-7599 E-mail: robert.coovert@exeloncorp.com | Mr. Jean Dubouis Liaison Engineer at INPO Electricite de France 602 Adams Lake Boulevard Atlanta, GA 30339 Phone: (770) 644-8752 Fax: (770) 644-8101 E-mail: dubouisj@inpo.org |

| Mr. Bob C. Evans Project Manager Nuclear Energy Institute Suite 400 1776 I Street, N.W. Washington, DC 20006-3708 Phone: (202) 739-8101 Fax: (202) 785-1898 E-mail: rce@nei.org | Mr. Jim Gallman (Team Co-Leader) Performance Analysis Supervisor TXU Comanche Peak PO Box 1002 Glen Rose, TX 76043-1002 Phone: (254) 897-5673 Fax: (254) 897-5977 E-mail: jgallma1@txu.com |
| --- | --- |
| Mr. Steve Garchow Human Performance Advocate Davis-Besse Nuclear Station 5501 N. State Route 2 Oak Harbor, OH 43449 Phone: (419) 321-8330 Fax: (419) 249-2320 E-mail: smgarchow@firstenergycorp.com | Mr. Jeffrey T. Gasser General Manager Southern Nuclear Operating Company Vogtle Electric Generating Plant P.O. Box 1600 Waynesboro, GA 30830 Phone: (706) 826-3139 Fax: (706) 826-3321 E-mail: jtgasser@southernco.com |
| Mr. J. Vincent Gilbert Project Manager, Performance Improvement Nuclear Energy Institute Suite 400 1776 I Street, N.W. Washington, DC 20006-3708 Phone: (202) 739-8138 Fax: (202) 785-1898 E-mail: jvg@nei.org | Christopher Hester Superintendent Operations Support Brunswick Steam Electric Plant 6024 Wrightsville Avenue Wilmington. NC 28403 Phone: (910) 457-2597 Fax: (910) 457-3369 E-mail: christopher.hester@pgnmail.com |
| Mr. Thomas C. Houghton Nuclear Energy Institute Suite 400 1776 I Street, N.W. Washington, DC 20006-3708 Phone: (202) 739-8107 Fax: (202) 785-1898 E-mail: tch@nei.org | Ms Karen S. Jennings Organizational PerformanceImprovement Manager American Electric Power Donald C. Cook Nuclear Station One Cook Place Mail Zone 11 Bridgman, MI 49106 Phone: (616) 466-3350 Fax: (616) 466-3377 E-mail: ksjennings@aep.com |

| Mr. John K. Kellams Supervisor, Human Performance and Causal Analysis Omaha Public Power District Fort Calhoun Station, FC 1-1 P.O. Box 399 Fort Calhoun, NE 68023-0399 Phone: (402) 533-6640 Fax: (402) 533-6747 E-mail: jkellams@oppd.com | Ms. Peggy Lucky Human Performance Manager LaSalle Generating Station 2601 North 21st Road Marsaeilles, IL 61341 Phone: (815) 357-6761 X2175 Fax: (815) 357-8333 E-mail: peggy.lucky@exeloncorp.com |
| --- | --- |
| Mr. Bruce C. MacKenzie In-House and Industry Operating Experience Consolidated Edison Company of New York, Inc. Broadway & Bleakley Buchanan, NY 10511-1099 Phone: (914) 271-7425 Fax: (914) 271-7291 E-mail: mackenzieb@coned.com | Mr. Terry L. Matlosz Manager, Organizational Development and Performance South Carolina Electric & Gas Company V.C. Summer Nuclear Training Center Highway 213, P.O. Box 88 Jenkinsville, SC 29065 Phone: (803) 931-5100 Fax: (803) 540-9208 E-mail: tmatlosz@scana.com |
| Ms. Caroline M. McAndrews (Team Co-Leader) Project Manager, Programs and Assessments Southern California Edison Company San Onofre Nuclear Generating Station P.O. Box 128 (D4B) San Clemente, CA 92674 Phone: (949) 368-9307 Fax: (949) 368-5195 E-mail: mcandrcm@songs.sce.com | Mr. Vernon J. McGaffic Assistant Team Manager Institute of Nuclear Power Operations Suite 100 700 Galleria Parkway, S.E. Atlanta, GA 30339-5957 Phone: (770) 644-8689 Fax: (770) 644-8549 E-mail: mcgafficvj@inpo.org |
| Mr. Walt McNeil Program Manager Human Performance The Detroit Edison Company 6400 North Dixie Highway Newport, MI 48166 Phone: (734) 586-4186 Fax: (734) 586-1883 E-mail: mcneilw@dteenergy.com | Mr. Tony M. Muschara Principal Program Manager, Human Performance Institute of Nuclear Power Operations Suite 100 700 Galleria Parkway, SE Atlanta, GA 30339 Phone: (770) 644-8677 Fax: (770) 644-8122 E-mail: muscharatm@inpo.org |

| Mr. Thomas J. Niessen, Jr. Assistant Plant Manager Browns Ferry Nuclear Plant Post Office Box 2000 POB 2C Decatur, AL 35609-2000 Phone: (256) 729-2044 Fax: (256) 729-2668 E-mail: tjniessen@tva.gov | Mr. William G. Noll Operations Manager Carolina Power & Light Company Brunswick Nuclear Plant Highway 87, P.O. Box 10429 Southport, NC 28461-0429 Phone: (910) 457-3059 Fax: (910) 457-3369 E-mail: bill.noll@pgnmail.com |
| --- | --- |
| Mr. Patrick P. Salkeld Human Performance and Self-Assessment Manager Watts Bar Nuclear Nuclear Plant P.O. Box 2000 Spring City, TN 37381 Phone: (423) 365-3635 Fax: (423) 365-1977 E-mail: ppsalkeld@tva.gov | Mr. Clark. D. Stafford Senior Evaluator Institute of Nuclear Power Operations Suite 100 700 Galleria Parkway, SE Atlanta, GA 30339 Phone: (770) 644-8483 Fax: (770) 644-8122 E-mail: staffordcd@inpo.org |
| Mr. Edward C. Sterling Department Leader, Nuclear Assurance Arizona Public Service Company P.O. Box 52034, MS 7964 Palo Verde NGS Phoenix, AZ 85072-2034 Phone: (623) 393-5670 Fax: (623) 393-5379 E-mail: esterlin@apsc.com | Mr. John C. Summers Senior Team Manager Institute of Nuclear Power Operations Suite 100 700 Galleria Parkway, SE Atlanta, GA 30339 Phone: (770) 644-8730 Fax: (770) 644-8549 E-mail: summersjc@inpo.org |
| Mr. David M. Ziebell Manager, Human Performance Technology EPRI 1300 West W.T. Harris Boulevard Charlotte, NC 28262 Phone: (704) 547-6107 Fax: (704) 547-6168 E-mail: dziebell@epri.com |  |

APPENDIX D

Change Management

Site: Brunswick							Process Map: 1.3

Description

The implementation of successful change is a fundamental criterion of a successful human performance program.  Identifying the full criteria in a change management process eliminates the potential for human errors and organizational weaknesses.

An extensive manual has been developed to help guide individuals involved with using a site approved change management model.  This model considers four major aspects for successful change:

- Assessment – Preparing For Change

- Communication – Understanding Change

- Affiliation – Embracing Change

- Capability – Enabling Change

Using methods and worksheet guides, full implementation of this change process results in the development of detailed change plans that guide individuals within the organization through the steps necessary for successful change management.  This formal process includes criteria used to determine when full use of the process is desirable.  The criteria include assessing the likelihood for disrupting day-to-day work routine expectations, alteration of personnel roles and responsibilities, and impact on how personnel are measured or compensated.

Enablers and Drivers

Successful implementation is accomplished through the assignment of a change leader or project manager responsible for applying the various steps outlined by the program.  At various phases of the development process, specific review and approval methods are used to ensure that the project manager is accountable for implementation of the process and development of a final change management plan.

Cost and Performance Measures

Costs associated with this program depend upon the extent of the change being implemented.  Full implementation is not always necessary; however, when utilized, costs would include project manager time and project team member time for data collection, plan development, review and approval and implementation.  Implementation costs may include document publishing and training costs dependent upon the nature of the final plan.

A recent example of the application of this process was the successful implementation of a major software change to the work control process that effected several established programs.  The process helped to identify such items as; the extent of benchmarking, communication, training, and additional software support requirements necessary to coordinate the change effectively.

APPENDIX E

Human Performance Continuing Training

Site: Brunswick						Process Map Area: 3.4

Description

Human Performance Workout Days are an integral part of the human performance continuing training program and are effective in communicating human performance issues and lessons learned to the station staff.  The training topics are determined by the Human Performance Leadership Steering Committee based on a variety of inputs including station trends, upcoming major goals for the station (such as an outage), and management intuition.  Once the topics are determined they are assigned to the part of the organization that has the most ownership of the topic.  Typically, one or two individuals are assigned the responsibility of developing and presenting the training to the entire staff, including contractors.  The training sessions are generally targeted for one to two hours and are scheduled over a two-week period each quarter to allow everyone to attend.  Training effectiveness has been enhanced through the use of various experiential training formats such as game shows, contests, and case studies using video clips from movies, plays, and others.  Employee feedback has been very favorable and the sessions are generally viewed as a positive part of the station's human performance program.  The management team fully supports the training.

Enablers and Drivers

This training program has evolved over time to its current format due to a variety of enablers and drivers.  Senior management at the station recognized the need to improve human performance and as an enabler, established a program to support that goal.  Initially, talking papers were distributed throughout the organization and the individual supervisor and/or manager would discuss the topic with their part of the organization.  Over time, it was recognized the presentations were not consistent due to the individual presenter and the time available.  Additionally, the training was generally viewed as ineffective by the station staff.  This led to the involvement of the Human Performance Steering Committee, formal scheduling, training format changes, and consistent presentations.

Cost and Performance Measures

The cost of this program is approximately eight hours per year per employee as well as the time needed to develop and present the training.  Other costs are variable depending on the format and have included expenditures for items such as videos, props, actors, etc.  Its effectiveness is measured, in part, by the reduction in errors after training is completed.

APPENDIX F

Pre-Job Briefing Databases

Site: Brunswick and Palo Verde					Process Map: 3.4, 3.5

Description:

A good practice was identified at two sites where pre-job briefing databases provide operators with the information necessary to conduct a comprehensive briefing.  In addition, these databases provide the vehicle for consistent information to be used by various work groups on specific activities conducted previously.  This information includes personnel required to conduct the task, procedure related information, and lessons learned from previous activities, internal operating experience, and industry operating experience related to the task.  In addition, the databases allow the operating crews to capture and store pertinent information learned during post job critiques enabling a subsequent crew to conduct the same evolution successfully in the future.

Palo Verde developed something that would be easy to use and provide the information needed to perform each job successfully.

The database will not enable improved performance without the culture of the operations personnel recognizing, gathering, and inputting pertinent operating experience.  Palo Verde operators and shift technical advisors routinely examine incoming industry experience for general lessons learned.  Tailboard evolution owners (assigned reactor operators and senior reactor operators) also look at the operating experience input for applicability to their database entries.  Operators perform post-job critiques for changes to practices or procedures, for key operating experience, and performance miscues to add to the operating experience section of the database.  The database is valuable because it gathers this operating experience in one central location. The database currently contains more than 200 pre-job briefs with operating experience, and is constantly growing.

Another key is the operations staff is diligent at looking at the database for upcoming evolutions, not only to prepare for the pre-job briefing, but also to anticipate any problems with the performance of the evolution.  The in-house operating experience is seen as useful in determining if a task can be successfully performed given an existing set of plant conditions.

The database provides an easy-to-use method to document pre- and post-job briefs as well as lessons learned for all levels of tasks.  The database also provides a link to the Institute of Nuclear Power Operations Web site.  If the database is missing operating experience, personnel can link to INPO’s Web site, find what is needed, and then cut and paste directly into the database for future reference.

Enablers and Drivers

This process is consistent with the station business plan to improve pre-job briefs at the site.  The methodology of using the front-line workers to develop the database drives the organization to become fully engaged in implementing various elements of the human performance strategy.

Cost Performance Measures

The program is free upon request of Palo Verde operations. To populate with data and maintain the data current, especially the in-house and industry operating experience, requires about 20 hours initially for each tailboard evolution and approximately ten hours per quarter to maintain. The performance indicator for this system is management observation of its use, feedback of anecdotal good catches, and the number of events for tailboard database evolutions. Developing the database has resulted in a number of improvements. During outages using this tool, the number of significant events has been reduced from three in 1998 to zero in 1999 and 2000.

APPENDIX G

Eliminating Latent Weaknesses in Jobsite Conditions

Site: Brunswick					Process Map Area: 1.3, 3.4, 3.7

Description

A dedicated effort has been initiated to identify and eliminate weaknesses in jobsite conditions at Brunswick.  This effort was initiated following a reactor scram that was the direct result of an institutionalized operator work around in a General Operating Procedure.  The site recommitted personnel and resources to the Scram Frequency Reduction Committee (SFR) with a focus on human performance fundamentals.  The SFR program has produced measurable near-term improvements in the existing program by identifying, documenting, and tracking the status of past technical recommendations made by the BWR Owners Group (BWROG).  Additionally, the committee has systematically interviewed personnel to gain insight into potential SFR opportunities as well as reviewing plant procedures and systems for potential single point and other scram vulnerabilities and latent organizational weaknesses.  A “Human Performance Workout Day” was dedicated to this initiative.  This training session emphasized the individual's role in eliminating scrams and the site’s dedicated effort in eliminating latent organizational weaknesses.  This effort has now been expanded into a review of design modifications and procedure changes currently being developed for an extended power uprate program.

Data from BWROG scram reduction recommendations and previous single point failure vulnerability studies were reviewed and yielded the following results:

- Identified the following plant modifications (Engineering Service Requests) for the recent Unit 2 Outage (B215R1):

- Eliminated single point scram vulnerability Unit 2 MSR Drain Tank High Level Turbine Trip Logic

- Eliminated MT Exhaust High Temperature trip

- Eliminated RFP Min Flow Valve Time Delay

- In conjunction with the RFP Reliability initiative:

- Installed Roller Bearings on RFPT Torque arms

- Replaced Unit 2 RFP Overspeed Trip Lockout Switch

- Identified these modifications for evaluation and possible implementation under the Maintenance Rule Key/(a)(1) equipment reliability process:

- Remove Low Vacuum Group 1 Isolation

- Lower the RPV low water level scram setpoint

- Eliminated CWIP trip on High Condenser Bay inlet pit level

- Eliminated CWIP trip on low Lube Water flow & replace with annunciation

- Install fault tolerant logic & control for turbine trip signals

- The SFRC used proactive benchmarking with other utilities to reduce the station’s susceptibility to reactor scrams. One example was to eliminate unnecessary 1/2 scram conditions during Reactor Protective system (RPS) system routine testing.

- The SFRC re-established a site wide effort pertaining to Scram Reduction by:

- Developed innovative SFR “Level of Awareness” training booklets. Gave presentations to all site groups based on the booklets.

- Developed an SFRC Inquiry process to submit scram frequency reduction.

- Developed an SFRC Web site.

- Initiated SFR customer site survey.  Received very positive results.

- Integrated the SFR program as a focus area with Human Performance Week (Since human performance events were historically the leading cause of unplanned scrams.)

- Site management representatives gave a presentation on Scram Reduction

- Video presentations and TWIN analysis

- “Even good alligators have a bad day”

- Piper Alpha oil platform disaster

- The review process has been integrated into procedure changes for latent weaknesses in jobsite conditions.  This review resulted in several procedure enhancements that were identified as institutionalized operator workarounds and annunciator response procedures.

Enablers and Drivers

The primary driver for this initiate is the elimination of unnecessary reactor scrams and re-visit past decisions regarding system designs and single point scram points.  Additionally, maintenance and operating procedures were reviewed for latent organizational errors and enhancements.  This initiative has resulted in the reduction of errors that could result in plant events and loss of generation capability.  The SFR committee is also in the review process for the modifications associated with the current extended power uprate project.

Cost and Performance Measures

Costs associated with this process have been the use of existing plant personnel in the review of procedures and systems designs.  The initial dedication of a multi-disciplined team consisting of operators, I&C technicians, engineering, and the scheduling group required several weeks. Additional costs are associated with the support of the BWR Owners Group Scram Frequency Reduction initiative.  The current costs are associated with the team reviewing design changes on a level of effort basis.

APPENDIX H

Common Precursors to Errors Report

Site: Comanche Peak					Process Map Area: 3.6, 4.1

Description

Common precursors to low-level events from the previous outage are summarized and fed back to first line supervisors and managers in an effective two-page report at the start of the current outage.

While an outage is progressing, as part of daily review of condition reports, the root cause consulting engineer selects condition reports with human performance implications for additional review.  Error likely situations are usually captured within twenty-four hours after the condition report is generated.  This is done in an interview with the personnel involved using a checklist.  Before the next outage, a pareto analysis is performed and a two page report is generated listing the most frequent error likely situations that may have been encountered as well as a paragraph providing actions personnel can take to eliminate or mitigate the error likely situation.

This report is provided to supervisors and managers as a tool to focus personnel on the important error likely situations they need to respond to.  Workers and supervisors state that the report provides the basis for pre-outage human performance discussions at the work group level.

Enablers and Drivers

This process is consistent with the station's goal of training all personnel in human performance fundamentals, and provides reinforcement of this key principle.  In particular, the focus on precursors (rather than event types) is clearly relevant to the job site environments and the work being conducted.

Cost and Performance Measures

The data collection and report preparation take approximately one man-month per outage.  The concise report is valuable for providing topical discussion points during site-wide human performance discussions that are held prior to and at the midpoint of outages.  The report was reported to be useful for busy supervisors preparing for pre-job briefs.  Overall the process works well to reinforce human performance fundamentals training in the workplace.  Several field supervisors responded favorably when questioned about the report.

APPENDIX I

Peer-to-Peer Observations

Site: Comanche Peak	Process Map Area: 3.4, 3.6, 4.1

Description

A peer on peer observation program is used to identify and correct behavior that places employee safety at risk.  The program is employee driven.  Employees are provided observation training prior to performing any observations.  Management observations are not a part of the process.  Once trained, employees find opportunities to observe other employee work activities.  The program requires the observed employee to consent to being monitored for at-risk behaviors. Observed behaviors, both safe and at-risk, are documented, trended, and reported to management.  Management recognizes and rewards safe behavior.  Approximately 3,000 at-risk behavior safety observations are performed yearly.

A bear symbol is prominently displayed all over the site, including a large billboard at the station entry indicating “You are Entering Bear Country.”  The BEAR acronym is defined as  “Behavior Equals Accident Reduction”.

Enablers and Drivers

Comanche Peak’s vice president, nuclear operations strongly supports this employee-driven process.  The BEAR program ensures confidentiality of participating employees. Greater than 99% of the employees consent to the observation when asked.

Cost and Performance Measures

Comanche Peak’s program was provided by a consultant (Behavior Science Technology), cost approximately $250,000, and took about one year to implement.  Performance is measured by comparing the number of at-risk behaviors to the total number of observed behaviors.

APPENDIX J

Plant Event Review Committee

Site: Comanche Peak	Process Map Area: 3.3, 4.1

Description

The Plant Event Review Committee (PERC) has proven to be an effective management process used to quickly perform an initial evaluation in response to any event involving human performance problems.  The PERC is defined in a station guideline and has been in use for over ten years.  The PERC, by definition, has four primary goals:

- Stabilize the situation

- Communicate relevant information to concerned parties

- Restore or repair damage, and

- A first step in preventing future similar events.

The PERC meets for an event as defined by the guideline or if requested by a member of the station's staff.  For a human performance related event, the membership would typically include the individuals directly involved with the event as well as the responsible supervisor(s) and manager(s).  It may also include individuals such as procedure writers, trainers, engineers or any other individual that may be able to contribute to understanding the event and preventing recurrence.  Any outcomes or required actions are documented using the station corrective action program.

Enablers and Drivers

The PERC process was adopted as a necessity in response to a series of significant events that occurred at the station in the early 1990s.  It was designed as a tool to expedite understanding and correcting the conditions that led to an event in order to prevent recurrence.  Initially, the preponderance of the work force viewed the PERC process as a negative and punitive management tool.  However, over time, management has achieved a high level of trust with the workers and consequently, the perception of the PERC review has become more positive.

Cost and Performance Measures

The cost of the PERC process is limited to the person hours required to serve on the committee and varies with the event being evaluated.  Station management views the PERC as one of the more important tools responsible for the significant reduction in human performance initiated events since the inception of this process.  Currently, it is not uncommon for individuals at the worker level of the organization to request a PERC in response to a human performance problem.

APPENDIX K

Weekly Human Error Investigation Process

Site: Farley					Process Map Area: 3.3, 3.4, 3.6, 3.7, 3.8

Description

The Human Performance Review Board (HPRB) is used to assist in developing corrective actions as well as providing a forum for management to demonstrate their commitment to the principle of non-disciplinary consequences for honest mistakes.  This process supports demonstration of management’s desire to promote teamwork, identification of organizational weaknesses, valuing prevention, and open communication.

Weekly, the HPRB selects an issue identified through the corrective action system as a focus event for the board to review.  Discussion focus is placed on the human performance and organizational, or programmatic factors involved in the issue.

The HPRB normally meets once per week and is composed of the plant general manager, the two assistant general plant managers, a licensing representative, a safety audit engineering review representative, individuals involved in the selected human performance issue and their supervisor.  The board is grounded upon the INPO “Excellence in Human Performance” principles.  It was directed to be a non-punitive system that encourages open discussion about the event.  Board discussion includes a sequence of event review, followed by a discussion of what factors may have led to any committed error(s).

Enablers and Drivers

This process is run as a non-punitive system to enable the plant to understand the underlying causes for events.  It is encouraged to be an open meeting where everyone involved can say what transpired and is encouraged to voice ways that related process can be corrected to prevent future events.  The site established this process because of a need to correct repeat errors, and a desire to change a culture that was previously considered punitive in nature.

Cost and Performance Measures

The costs involve preparation time for the meeting and its conduct.  Improvements resulting from this process include the identification and correction of organizational weaknesses, however, additional benefits are being seen in increased self-reporting rates and improved communication between management and frontline workers.

APPENDIX L

Performance Indicators

Site: Farley							Process Map Area: 3.6

Description

Southern Nuclear Company is working in conjunction with EPRI on a leading indicator project.  This project focuses on identifying those factors that predict future human performance and organizational health.  Those factors thought to most affect human performance were identified and trends were developed on both a station level and a departmental level.  Lagging indicators were also selected to validate the leading indicators.  As the leading indicators change, the lagging indicators are monitored to determine if the change in indicators that will best predict future human performance.  By trending those items that were thought to be important to human performance, the plant is able to determine actual progress and make changes accordingly.

For the last two years there has been the expectation for supervisory personnel to make one human performance observation per month.  When the plant began trending observations it was noted that only 0.2 observations per month per supervisor were being performed.  Now that observations are trended, the number has increased too greater than one per month.

The experts that study human performance indicate 80-90% of all errors are tied to the organization.  The plant started trending corrective actions to determine what percentage of corrective actions were “coach the individual” (negative reinforcement) versus correct the process.  It is easy to coach individuals and close out condition reports but harder to find the organizational problems that led the individuals to make the errors.  Initial trending efforts indicated that coaching was the corrective action in 45% to 50% of the cases.  Indicators show that the plant is now correcting processes at a rate of 85% to 90% and coaching the individuals in 10% to 15% of the cases.  Other items important to human performance have improved as well.

Enablers and Drivers

EPRI and the station's management are working together to make this program successful.  The Licensing Group coordinates the project with inputs from the various plant departments.  The trends are produced and evaluated monthly.

Cost and Performance Measures

Cost is based on man-hours to enter data into the database and to trend the results.  Leading and lagging indicator are monitored to determine desired results and goals.

Current Indicators:

- Awareness and Opacity (Human Performance Awareness)

- Human Performance Training

- Human Performance Supervisory Observations

- Preparedness (Backlog Management)

- Procedure Revision Requests

- Open Corrective Actions

- Just Culture and Top-Level Commitment (Workplace Culture)

- Negative Reinforcement as Corrective Action

- Self-Identification of Problems

- Overtime/Excess Straight Time Rate

Lagging Indicators

- Human Performance Error Rate

- Personnel Injury Rate

- Human Performance Significant Events Rate

Changes to be implemented soon are indicated below:

- Top-Level Commitment

- Overtime/Excess Straight Time Work Hours

- Executive Visits to Plant

- Awareness

- Human Performance Training

- Work-arounds

- Preparedness

- Procedure Revision Requests

- Open Corrective Actions

- Planned to Unplanned (emergent) Work

- Just Culture

- People Centered Reinforcement

- Self-Identification of Problems

- Turnover

- Absenteeism

- Learning Culture

- Formal Self-Assessments

- Benchmarking Trips

- Assist Visits

- Opacity/Transparency

- Human Performance Supervisory Observations

- Lagging Indicators (Note:  Contractors to be tracked separately)

- Human Performance Error Rate

- Personnel Injury Rate

- Event Free Days.

APPENDIX M

Management Observation Training Program

Site: Farley	Process Map Area: 3.1, 3.3, 3.4, 3.6

Description

A two-day human performance observation course was taught to all supervisory personnel (foreman and above) and a significant number of the engineers.  This course included leadership topics, human performance fundamentals and tools, error precursors, observation process, observation techniques, confronting unwanted behaviors, practical exercises, actual observations, and sharing of lessons learned.  Emphasis is placed on how to confront unwanted behavior in a non-punitive matter.  During an observation the observer will identify unwanted behavior and discuss both the behavior with the observed employee and the corrective action needed.  The supervisor works with the employee immediately to get buy in and a verbal commitment to improve that behavior.

Individuals required to perform observations are tracked to ensure the observations occur as required.  The results for the plant and each group are trended and the strengths, areas for improvement, and significant comments and recommendations are summarized.  An action item is entered in the action item database that requires each group to review their data and identify appropriate actions for group improvement. The licensing group monitors and analyzes the results of the observations.

Additionally, a one-day course was developed from the two-day course for presentation to worker level personnel.  This course has been presented to maintenance journeyman, system operators, and is scheduled for presentation to chemistry technicians, health physics technicians, the new system operators class, and to reactor operators this summer.

Enablers and Drivers

The training course provides the tools for supervisors to perform effective observations. This requires the supervisor to attend the two-day training class.  Additionally, plant personnel are provided information concerning the observation process and the expectations of managers concerning observations.

Cost and Performance Measures

The cost of this process is primarily the work hours required to attend the training and perform the observations.  The performance measures are the resulting trends developed from the observations

APPENDIX N

Health Physics Observation Program

Site: Farley	Process Map Area: 3.5, 3.6

Description

The purpose of the program is to determine trends in radiation worker practices in an effort to improve those practices and to provide the training department and management with tools to help their workers succeed.

The station's health physics technicians implement the program.  These personnel are assigned strictly to observe radiation workers and provide on-the spot feedback for good practices as well as those that need improvement.

Data is collected on a Health Physics Observation (HPO) Form.  The data from this form is collected in a computer program that outputs the collected data in both word and graphical formats.  This information is then reviewed to determine if there are trends developing within a work group or for the plant as a whole in order to stop bad habits from taking root.  A monthly report is distributed to managers, superintendents and supervisors to share with their personnel

Enablers and Drivers

The HPO program was developed as a more “worker friendly” replacement for the Radiation Incident Report.  Since no names are mentioned, workers are less intimidated by the observations and they feel freer to express comments and questions regarding health physics and radiation worker practices.  This has proven to be a very positive program as several groups have requested health physics to share the data for a group with them at safety meetings and other group gatherings.  The data is presented and the health physics representative shares methods to reduce the errors as well as sharing the positive aspects of the group’s observation data.

Cost and Performance Measures

The cost for this program is the time spent by a health physics technician and time spent entering and analyzing data and it is included in department budgets.

The results are listed below:

- Radiation worker errors have declined an average of 3-5% per year since the program’s inception in 1997.  This translates into fewer personnel contamination events, fewer contaminated areas in the plant and a reduction in new contaminated areas. Also, less radioactive waste is generated and clothing to be washed results in reduction in manpower invested to decontaminate an area due to careless radiation worker habits.

- Reduced radiation exposures due to heightened worker awareness of low dose waiting areas (workers are questioned as to where their nearest low dose waiting area is located).

- Percent difference between DADs and TLDs is closer than the industry average

- Better and more positive working relationships between health physics and other plant groups.

APPENDIX O

"Focus on Five" Prevent Event Briefing Questions

Site: Palo Verde	Process Map Area: 3.5

Description

“Focus on Five” is one of the strategies Palo Verde has implemented to drive home use of error reduction and event prevention techniques before performing any station tasks.  The key to this strategy is focusing the staff on the task at hand with a simple, easy-to-understand tool.

Before starting any task, from simple to complex, employees are expected to ask themselves the following five questions:

- What is the task?  Do I understand it?

- What is the worst thing that can happen and how can I prevent it?

- What else could go wrong?

- What are the safety and/or radiation protection considerations?

- Are my training and qualifications up to date for this task?

By making these questions the focal point of pre-job briefs and facilitating discussion about the upcoming task, workers are better focused on the task at hand and can more readily identify issues that could cause an event.  Participation by all personnel involved in a task is an essential ingredient to success.  This tool is so flexible that an individual can use it before performing back shift work without the benefit of a leader and reduce the potential for error and eliminate the potential for an event.

To implement this effort, input was requested across the site on developing appropriate questions.  Questions were developed and revised twice before issuing for use.  Laminated cards with the prevent event questions on one side and basic safety expectations on the other were designed and produced by the site safety committee.  Issuing the safety expectations on the back of all cards was seen as a way to reinforce/remind personnel of methods to achieve event-free performance.  For example, one of the safety expectations listed is that workers perform a “two-minute drill” at the job location prior to work and after any extended break in job performance.  This two-minute drill helps workers identify whether the task and the conditions under which the task is to be performed are as anticipated.

Initial roll-out of the tool generated many “good-catch” examples.  To take advantage of these examples and maintain focus on the tool, a video tape was developed to share lessons learned.  In the tape, a senior manager performed various parodies to reinforce, in a humorous manner, that errors and events can happen anywhere and anytime.  Additionally, plant personnel were video taped providing specific examples of how use of the tool helped workers avoid error, injury and even plant events.

A comprehensive, interactive communications plan is being implemented.  Using a combination of videos, cartoons, puzzles and articles, workers see weekly how they can employ these strategies reduce the potential for error and the prevention of events.

Enablers and Drivers

Palo Verde senior managers and leaders play an integral part in anchoring this strategy.  They rolled out the pre-job brief and two-minute drill expectations to their staff members as well as distributing the lanyard-ready cards to site personnel.  Leaders follow up by observing employees to be sure they are using the cards and questions prior to work.  Senior managers observe the use of pre-job briefs and two-minute drills in their respective organizations and provide feedback success to the leaders.  Based on their feedback, Palo Verde’s Prevent Events steering committee considers developing additional actions for continuing performance improvement.

This program is effective due to the well-defined expectations associated with the tool and its expected use on every task.  Leaders consistently reinforce its use.  Good examples of its use are documented and are frequently incorporated into communications tools for reinforcement.

Cost and Performance Measures

The “Focus on Five” lanyard cards were produced for all station personnel for about $500.  Additional media costs for posters and publications totaled approximately $500.  A performance measure is observing the improvements in performance over time, especially examples that can be traced back to a "Focus on Five" brief.  Since implementation of "Focus on Five" in 2000, Palo Verde has experienced only 3 significant human performance events as compared with 14 in 1999.  A declining trend of lower-level human performance events has occurred during the  third and fourth quarters of 2000.  These promising results demonstrate that a simple tool, well understood by the workforce, can have a significant positive effect on human performance

APPENDIX P

Hands-On Safety Behavior Laboratory

Site: Palo Verde	Process Map Area: 3.4

Description

Palo Verde workers are given the opportunity to develop hazard recognition skills in a controlled, safe environment under the tutelage of a safety professional.  The station has accomplished this task through the marriage of laboratory/computer-based instruction and hands-on applications instruction in a “Safe Behavior Workshop.”

Topics in this laboratory include housekeeping, ergonomics, ladder safety, foreign material exclusion, scaffolds, fall protection, personal protective equipment, hearing conservation, hazard communication, fire and electrical safety, and prevent event tool recognition.  When groups use the laboratory, a set of questions that can only be answered by experiencing the various stations is given to teams of three to four individuals.  This forces them to interact with each other at a given station and discuss what they see.

One key to the value of the “Safe Behavior Workshop” is the student’s interaction with the safety professionals.  The side benefit for safety is that the employees learn to think of the safety professionals as a subject matter resource for consulting use in the field rather than “safety cops” writing people up for violating proper safety behaviors.  The safety professionals have accumulated Palo Verde operating experience examples for use in the workshop to show what has gone wrong in the past and to make it real for the students by hands on interactions with the devices, tools and/or protective devices.  By being right there with the student, the safety professional can answer questions, provide coaching to experienced workers, and show appropriate behaviors to the uninitiated workers.

Personnel well versed in hazards recognition and control are much less susceptible to personal injury.  Palo Verde firmly believes in the philosophy of "Do it Once-Do it Right."  The “Safe Behavior Workshop” supports this philosophy by equipping workers to plan safe work activities. Well-equipped workers are more productive and experience fewer injuries.

Enablers and Drivers

Senior management and line management provide support for all employees experiencing the workshop prior to each outage and with two outages a year that means twice every year.  The experience is impressive enough that some groups are going through the workshop based on word of mouth recommendations by their peers.

Cost and Performance Measures

The poles and curtains for the booths and the various materials used to demonstrate equipment uses and traps, such as ladders, drills, blue-bells, and fall protection systems, cost about $8,000. In addition a room of approximately 1,500 square feet is required to house all the booths. The management trend report measuring recordable and lost time injuries per year is the performance measure.  Palo Verde also measures safe and event prevention behaviors through a management observation program.  Since putting the laboratory into place, the year 2000 had the second best safety results in Palo Verde history.

APPENDIX Q

Human Performance Marketing

Site: Palo Verde	Process Map Area: 1.4, 3.1, 3.5

Description

In late 1999, recognizing a need to put a greater focus on the prevention of plant events and personal injuries, Palo Verde management developed a formal Prevent Events program.  Part of that program was a focused communications campaign that included video, posters, newsletters and cartoons.  That campaign started with the production of a video in which Palo Verde’s three vice presidents described the purpose of the program and its major components — five prevent event questions, pre-job brief and two-minute drill — and set the expectations for their use for all jobs.  This video included examples of how to use the tools in the field and in an office setting. As a result the station employees received written tools use expectations, verbal reinforcement of the expectations, and finally a video representation of the tool use expectations.

Several months later, this video was followed with one hosted by the director, site maintenance, who infused humor into his introductions by vignettes that illustrated the success workers had when they used each of the five Prevent Events questions.

In June 2000, the video campaign was expanded to a full communications campaign that included:

- Weekly information in the Palo Verde 2001 newspaper in a Prevent Events box describing a tool or a success using a tool, as well as other general description articles.

- Cartoons that illustrated actual Prevent Events successes at the plant and examples of what could go wrong by not using the tools.

- Prevent Events puzzles and  crosswords

- Information and examples on Vista Vision 2000 TV monitors

- A variety of posters and elevator placards that carried diverse Prevent Events messages

A subsequent video used characters Gunner Gethurt (who always gets hurt because he doesn’t take time to use the tools), Noah It’All (who thinks he knows everything and doesn’t need the tools) and Karen F D’Act (who carried the Care Enough to Act message and intervened to ensure Gunner and Noah completed their job without incident).

Prior to the fall 2000 outage, another video was produced for contractors who would work in Palo Verde Unit 2’s refueling outage.  Using portions of the first video produced, it explained the program and its components to outage contractors and clearly set plant management’s expectation for use of the tools.

At the end of 2000, a self-assessment of the entire Prevent Events program was performed.  The assessment determined that employees had an excellent understanding of the strategy and tools available.

Enablers and Drivers

The initial video was required viewing.  After that, word of mouth endorsement of the product led to continued viewing of subsequent videos.  The workforce liked the idea of being able to visualize what the expectations look like in practice.  In addition, the use of posters, newsletters cartoons, etc., engages personnel at all levels.  Awards are made out of the cartoon originals, signed by the vice president, nuclear production, for submitting a cartoon idea.

The communications group is also a part of the Prevent Events strategy.  The group also has an observation checklist, error prevention tools and expectations consistent with the site strategy.

Cost and Performance Measures

If a plant has no video capability, the cost would include camera and computer aided editing equipment, which can cost around $800 to $1000.  With existing video equipment, the cost is the time of employees to participate in the production. Also required is a person on staff who can story board and produce video vignettes.  Palo Verde has a staff of eight people in the communications group with a variety of production skills.  A recent human performance self-assessment indicated an excellent recognition by employees of the Prevent Events strategy and tools.  Palo Verde had its fewest-ever significant human performance events in 2000 — three — compared to 14 the previous year.  OSHA recordable injuries were limited to 14, Palo Verde’s second-best-ever performance, and down from 17 in 1999

APPENDIX R

Quarterly Human Performance Ownership and Effectiveness Measures

Site: Seabrook						Process Map Area: 3.4, 3.6, 3.7, 3.8

Description

The department human performance coordinator and the respective department manager submit a quarterly “ownership and effectiveness measurement” report to the station human performance manager. The measurement is a survey that grades human performance improvement effort for the particular department and is reported to the department’s human performance review committee.  The survey prompts managers and department personnel to focus on human performance issues and activities important to successful performance improvement.

Several areas and issues are included in the computation of the overall grade.  These items include:

- Observation quantity and quality goals

- Condition report self-reporting

- Department attendance at required human performance training

- Self assessments of human performance

- Identification of opportunities to improve

- Plans to improve areas needing improvement

The measurement promotes an added level of accountability for department managers.  Additionally, preparation of the report gives the manager an up-to-date awareness of the overall state of human performance within his or her area of responsibility.

Enablers and Drivers

The requirement for the quarterly report is instituted in a Seabrook Station Administrative Procedure.  Senior station management expects all departments will remain attentive to human performance and areas needing improvement.  The respective department’s human performance review committee reviews this report on a quarterly basis.

Cost and Performance Measures

Preparation of the ownership and effectiveness measure report requires about one to two hours by the department human performance coordinator.  The respective department manager, after reviewing the report, signs it indicating his approval.

APPENDIX S

Organizational Communications

Site: Seabrook						Process Map Area: 3.2, 3.3

Description

Daily communications practices are effective in establishing and maintaining organizational alignment on station priorities as well as distributing information to all employees.  Each day a meeting is held at 6:15 a.m. and it is attended by personnel from across the organization.  This meeting reviews the priorities for the day included the “protected train”, site events, risk level, and other information of interest.  The individual representatives then return to their work group and relay this information at a 7:00 a.m. meeting.  In some cases, due to the physical location of the work group, the meeting package and notes taken by the attendee are faxed back to the work group and another individual leads the discussion.

Additionally, the operations manager broadcasts a message throughout the organization via the phone mail system after the 8:00 a.m. morning managers meeting to communicate information of interest to the organization.  At a higher level, the plant manager routinely sends e-mails to site employees on topics of interest once or twice a week to keep them apprised of station and/or company activities.  He also routinely sits in on the 7:00 a.m. morning communication meetings of different groups to critique meeting and to answer any questions or concerns the employees may have.  Employee feedback indicates this practice has been effective in quelling rumors and improving organizational teamwork.

Enablers and Drivers

The improvements in communications have come about as a direct result of a station management commitment to improve teamwork and morale at the station.  This commitment was the result of some important station events, feedback from external organizations, and feedback from employees.  Individuals at all levels of the organization, including contractors, cited improved communications and management approachability to answer questions and concerns as two of the keys to improving the station's human performance record.

Cost and Performance Measures

The cost of this process is comprised of the employee time to attend the meetings in each part of the organization.  Each daily meeting lasts approximately 15-20 minutes.  Members of the management team attend meetings throughout the organization on a random basis and provide performance-based feedback to the individual work group.

APPENDIX T

DeskSTAR

Site: Seabrook							Process Map Area:  3.1, 3.4

Description

The software program contains two different simulators, a pump & valve control panel and an electrical panel.  The simulator guides the participant through three scenarios on each panel.  Following each scenario the user is provided performance feedback.  In addition, to improve the learning experience, the user's DeskSTAR performance is tied to the performance of the fictitious nuclear generating facility.  The INPO rating, stock price, injuries, NRC regulatory margin and public perception change after each scenario are forecast based on the user’s performance.

The DeskSTAR tool was implemented prior to the last refueling outage to raise plant personnel awareness in how to reduce human performance issues during the outage.

Enablers and Drivers

This computer program was developed to increase the awareness and future performance of station personnel with regard to self-checking, procedure use and adherence, peer checking, and procedure place keeping.

Cost and Performance Measures

The cost of the program consisted of the individual time for application development and employee time to complete the module.  Industry cost for the software is minimal due to Seabrook’s willingness to share it with other commercial nuclear generating stations.  Performance measures include the normal corrective action program trending for self-checking, procedure use and adherence, and other checks that to monitor the effectiveness of the computer application.  Additional costs include approximately two hours per employee to complete the module.  Seabrook also chose to provide a human performance shirt to those employees attaining a perfect simulation score.

APPENDIX U

Excellence in Performance Program

Site:	Watts Bar						Process Map Area: 3.4, 3.6

Description

Watts Bar has implemented a structured approach for understanding performance expectations of operational excellence with the “Excellence in Performance Program (EIP).”  EIP provides detailed performance expectations, a method for individuals to evaluate their knowledge, a means to measure and provide feedback, with an emphasis on coaching day-to-day conduct of key processes, including human performance.

Individuals are encouraged to perform a computer-based self-evaluation of their knowledge of key processes and performance expectations each week.  The EIP knowledge module consists of a large bank of situational questions that challenge the workers’ knowledge of individual human performance techniques and station standards such as self-checking, three-way communication, and independent verification.  These questions were generated by employees from within the group.  The program reinforces correct responses and immediately tutors the worker following an incorrect response.  Each worker is expected to take a 10-to-20 question computer-based examination on a weekly basis.  The data obtained during the computer-based examination provides indicators to the site on performance areas that may require additional attention by the management staff.

In addition, supervisors are encouraged to observe employees in the field, provide coaching as appropriate and document results for overall improvements in the organization.  Every week, the program supervisor trends the results from the computer-based examination and the information are provided to the department managers for sharing with the organization.

Enablers and Drivers

The Watts Bar vice president, plant manager, and direct reports strongly support this program.  Employees also support and participate in the program with the computer-based self-evaluation process.  Managers and supervisors are in alignment with the standards, expectations, and are committed to improving all aspects of the organization, including a strong focus on human performance improvement.

Cost and Performance Measures

Watts Bar’s program was provided by a consultant, cost approximately $200,000, and took about one year to implement, including building the data base of questions and training site personnel.

APPENDIX V

STAR 7

Site:	Watts Bar	                                 Process Map Area:  3.1, 3.2, 3.3, 3.4, 3.6, 3.7

Description

Watts Bar program has implemented the “STAR 7” program (Strategic Teamwork for Action and Results) along with TVA’s “seven values.”  The seven values are:

- Integrity

- Respect for the Individual

- Accountability

- Teamwork

- Innovation and Continuous Improvement

- Honest Communication

- Flexibility.

Watts Bar established “STAR 7 Winning Behaviors” that prescribe the behavioral framework for implementing the core values.  All employees have received training on these values.  Additionally, supervisory and management training ensures that management has the skills and tools necessary for implementing the core values.  There is strong involvement at the executive level in the training sessions and at the sites.  For example, the site vice president facilitates modules on the STAR 7 values and attends meetings to listen for how these values have become part of normal operations.

Feedback is solicited at the individual level and programmatic level.  The Values and Winning Behaviors are an integral part of quarterly and annual employee performance appraisals.  Management receives 360-degree feedback from employee surveys. Results from STAR 7 program surveys of employees are used to continually improve the program.  STAR 7 posters, table tents, note pads and other materials are used frequently to give continuing visibility to STAR 7.

Enablers and Drivers

Watts Bar vice president, plant manager, and direct reports strongly support this program.  This program has been rolled out to the entire organization, with reinforcement training provided to various levels of the management team.  This is part of the management commitment to improve existing management/supervisory practices.  Consultants were brought in to improve the ability of organizations to work collaboratively, reduce organizational barriers and improve teamwork.

Cost and Performance Measures

Watts Bar’s program was obtained from a consultant and implemented by trained Watts Bar facilitators.  It took about a year to implement, which included training all site personnel.  Continuing training is also provided on an ongoing basis.  STAR 7 is a structured program that facilitates a culture change within the organization with strong ties to individual (manager) performance appraisals.  The program is driven by the management team and the performance is measured by the success of the overall organization.

APPENDIX W

Training Alignment to Improve Human Performance

Site:	Watts Bar					Process Map Area:  3.4, 3.7, 3.9

Description

Training is an active part of the overall organization in driving for excellence in human performance.  The operator training process has developed a method to assess the needs of each operating crew prior to their training week.  A training instructor is assigned to each crew to observe an individual crew and tailor training to the areas needing improvement, especially in human performance behaviors.  Examples of these behaviors include, three-way communication, peer checking, self-checking, procedure compliance, and place keeping.  The training instructor is assigned to spend approximately one to two back-shift tours with the operating crew per training cycle.

Enablers and Drivers

Identification of weaknesses in human performance behaviors led to the operations

and training managers developing expectations for training instructors to independently observe real-time plant behaviors and integrate the results into scheduled training the next week.

Cost and Performance Measures

Watts Bar’s program is an internal program, driven by the line ownership and department manager personal involvement.  Weak performance behaviors observed during real-time plant conditions are quickly corrected before they become engrained into the work practices of the individual and crew competencies.

APPENDIX X

Error-Likely Days

Site:	Davis-Besse					Process Map Area:  3.3, 3.5, 3.6

Description

Error-likely days are proving to be an effective tool at the Davis-Besse Nuclear Power Station in reducing the number of human performance events.

Several inputs are used in an attempt to predict days when the probability of an error-induced event is high.  These inputs include historical data from the day of the week and month event probabilities, work planned for the day, risk of the planned activities, upcoming holidays, company related distractions, as well as other recognized error precursors.  When these inputs collectively indicate a high-risk day from a human performance perspective, the management team declares that day to be an “Error-Likely-Day.”  This is then communicated to the plant staff through a highly visible traffic light located at the plant entrance turnstiles by illuminating the yellow light.  It is also briefed throughout the station at the morning “shop talks” and pre-job-briefings.  Additionally, a higher management/supervisory presence in the field is expected to observe and monitor on-going work.  Lastly, management reviews the higher risk work scheduled for the day in an effort to put additional barriers in place to lower human performance risk.

Enablers and Drivers

The station's management implemented the error-likely-day concept to increase the awareness and potential impact of predictable errors to the plant staff.  It is controlled and driven by the management team with inputs from various parts of the organization.  The enablers include recognizing, understanding, and predicting future events based on historical data and well as current conditions.

Cost and Performance Measures

The implementation cost of the error-likely-day concept has been minimal.  The only discrete cost has been approximately $150 for the cost of the human performance traffic light at the plant entrance.  Since implementation of this feature to the human performance program, the events, based on monitoring, have been reduced by more than half.

APPENDIX Y

Outage Initiatives

Site: LaSalle							Process Map Area:  3.3, 3.5

Description

LaSalle prepares for an outage with a proactive focus on human performance, communications, and industry safety.  A key overall message is to proactively promote proper behaviors during the outage.  The goal is “No human performance-related events.”  The station prepares for the outage by providing training for the entire organization on key fundamentals for performance such as peer checking, self-checking, flagging, and pre-job briefings.  In addition, an awareness/fair day is held with the focus on outage fundamentals, outage handbooks with key information for all workers, greeting the employees on special kick-off days, special communications packages focused on outage error-likely situations, and positive reinforcement dollars for use by all managers.  As an example, the positive reinforcement dollars program was developed to reward positive behaviors.  Managers and supervisors, as well as workers on a peer-to-peer basis, are expected to spend time in the field seeking positive behaviors.  When found, these behaviors are rewarded on the spot.  When a reward is provided, the proper behavior rewarded is documented on the outage dollar.  Over 5,000 individual rewards were provided in a 20-day period.

Enablers and Drivers

The LaSalle outage manager and plant manager sponsor the human performance High Impact Team for improving human performance, communications, and industrial safety.  The intent is to have everyone involved in improving behaviors.  The management team focuses on positive behaviors of the workforce and ensures human performance behaviors are observed and rewarded at all levels.  Everyone from the manager to the frontline worker are involved with the outage dollar program.

Cost and Performance Measures

The overall cost for LaSalle was approximately $40,000.  This included the outage awareness day training, outage books, and positive reinforcement gifts for the workforce.  The “Outage Dollar” program represented approximately $15,000 of the total cost.  The previous outage indicators identified at least two significant events (LERs) and 18 prompt investigations performed on human performance events only.  These initiatives implemented for outages have contributed to a significant indicator improvement with zero significant events and only two prompt investigations performed on low-level events.

APPENDIX Z

Glossary

Note:	This glossary is consistent with the glossary in INPO’s Human Performance Fundamentals Course Desk Reference as of the date of this report.  The Benchmarking Team has made substantial input to the Desk Reference during the course of benchmarking.  INPO will continue to improve the Desk Reference and it’s glossary.  Any future improvements will be available on-line at http://www.inpo.org .

| Active Error | Action (behavior) that changes equipment, system, or plant state triggering immediate undesired results |
| --- | --- |
| Alignment | A measure or judgement of the extent to which the values, processes, management, and existing factors within an organization influence human performance in a complementary and non-contradictory way. To “align” is to change the values, processes, management, and existing factors within an organization so that together they influence human performance in a complementary and non-contradictory way. E.g., “Behaviors at all levels need alignment to improve individual performance, reduce errors, and prevent events. Alignment involves facilitating organizational processes and values to support desired behavior.” (Excellence in Human Performance) |
| Anatomy of an Event | A cause-and-effect illustration of the active and latent origins (linkages) of plant events initiated by human action |
| Behavior | a.) Observable (movement, speech) and non-observable (perception, thought, decisions not to act or inaction, emotional response, etc) activity by an individual. Generally, the nuclear industry treats observable behavior as measurable and controllable. Frequently, in context behavior is equivalent to observable behavior. b.) The mental and physical efforts to perform a task (Excellence in Human Performance, 1997). |
| Benchmarking | The practice to identify beneficial practices, compare performance standards, and discover innovative thinking or approaches; a process of comparing products, processes, and practices against the toughest competitors or those companies recognized as industry leaders |
| Business Plan | The overall document that links station mission and strategic goals to everyday work processes and improvement initiatives. The business plan stimulates management commitment, accountability, and follow-up that are critical for station success. |
| Causal Assessment | The systematic process of determining fundamental explanations for performance gaps or adverse trends |
| Change Management | A methodical process that enables leadership to establish the direction of change, align people and resources (including motivation), and implement the selected modifications throughout the organization |
| - Continuous Improvement | Small incremental changes initiated by team members |
| - Culture | An organization’s system of commonly (communally) held values and beliefs that influence the attitudes, choices and behaviors of the individuals of the organization. (for example, see safety culture) |
| - Defense | A measure, including expected behavior, that protects against various hazards or mitigates the consequences of a hazard |
| Defense- in-depth | The physical plant’s and administrative system’s built-in capacity to detect or prevent errors without suffering undesirable consequences; that is the multiple functions and associated techniques that exist within the human performance system a) to protect people from error, and b) to protect the physical plant from people's actions “All safety activities, whether organizational, behavioral or equipment related, are subject to layers of overlapping provisions, so that if a failure were to occur it would be compensated for or corrected without causing harm to individuals or the public at large. … The principle of defense in depth is implemented primarily by means of a series of barriers which would in principle never be jeopardized, and which must be violated in turn before harm can occur to people or the environment. … Human aspects of defense in depth are brought into play to protect the integrity of the barriers, such as quality assurance, administrative controls, safety reviews, independent regulation, operating limits, personnel qualification and training, and safety culture.” INSAG-3 Revision 1(INSAG-12), “Basic Safety Principles for Nuclear Power Plants” page 17. |
| Error | An action that unintentionally departs from an expected behavior |
| Error Precursors | Unfavorable factors embedded in the job site that increase the chances of error during the performance of a specific task by a particular individual. Frequently grouped in the following sets: human nature, individual capabilities, task demands, and work environment. |
| Error-likely Situation | A work situation in which there is greater opportunity for error when performing a specific action or task due to error precursors. Also known as "error trap." |
| Error Rate | The number of errors over time usually normalized to number of hours worked. |
| Event | Human Performance-related sense: An unwanted, undesirable change in state of plant structures, systems, or components (SSC) or human/organizational conditions (health, behavior, administrative controls, environment, etc) that exceeds established significance criteria and that involves human action or inaction in the causal chain. General engineering sense: An unwanted, undesirable consequence to the safe operation of the plant due to a change in state of plant structures, systems, or components (generally in terms of reduced safety margin) (WANO Root Cause Analysis) From Excellence in Human Performance (1997): Events are undesirable consequences that challenge the safety of the reactor core.” |
| Expectations | Established explicit descriptions or implicit understandings of acceptable plant outcomes, business goals, process performance, safety performance, or behavior |
| Factor | An existing condition that influences behavior and that is the result from some prior process or action of other individuals. Factors can positively or adversely influence behavior. Factors that adversely influence behavior are known as error precursors. |
| Feedback | Information about past or present behavior and results that allows an individual or an organization the opportunity to change. |
| Flawed Defenses | Defects with administrative or physical defensive measures that, under the right circumstances, may: 1) fail to protect plant equipment or people against hazards; 2) fail to prevent the occurrence of active errors; 3) fail to mitigate the consequences of error |
| Gap Analysis | The process of comparison of actual results or behavior with expected results or behavior. |
| Human Performance | Individual sense: A series of behaviors executed to accomplish specific task objectives (results) (Excellence in Human Performance, 1997); Organizational sense: the sum of what people (individuals, leaders, managers) are doing and what people have done; the aggregate system of processes, influences, behaviors, and their ultimate results that eventually become manifest in the physical plant. Note that what some people have done affects what others will be doing later. A “result” for one person may be a “factor” for another. |
| Individual | An employee in any position in the organization, i.e., worker, supervisor, staff, manager, and executive |
| Job Site | The location where behavior takes place. Note: Every person in the organization acts in many different job sites at different times, and therefore is influenced by different factors each time action occurs. |
| Job Site Conditions | The unique factors associated with a specific task and a particular individual; factors embedded in the immediate job site environment that influence the behavior of the individual during work |
| Leader | An individual who takes personal responsibility for his or her performance and the plant's performance and attempts to influence the processes and values of others |
| Leadership | a.) The behavior of a leader attempting to influence the processes and values of others. b.) That group of employees given the responsibility for guiding the goals and values of the organization. |
| Leadership Practices | Techniques, methods, or behaviors used by leaders to guide, align, motivate, and inspire individuals relative to the organization’s vision (See practices) |
| Management | That group of employees given the responsibility for the performance of the organization. See also Leadership. |
| Mission | a.) The ultimate accomplishment for which the organization exists and which requires programs of action, for example, ‘generate electricity’ or ‘supply energy” |
| Organization | A group of individuals with a shared mission and set of processes to apply resources and to direct people's behavior toward safe and reliable operation |
| Organizational Factors | - Task-specific sense- an existing condition that influences behavior of a human being and that is the result from prior organizational processes. "Task demands," "work environment," and culture could be called 'organizational factors' in this narrow sense. - General sense- the aggregate of all management and leadership practices, values, culture, corporate structures, processes, technology, resources and controls that combine to result in the currently existing conditions which affect behavior of individuals at the job site. |
| Performance Gap | The difference between desired performance and actual performance, whether in terms of results or in terms of behavior. |
| Performance Indicator | A measure of performance (after the fact) using quantitative parameters that provide information on the condition or direction of key result areas (also known as metric) |
| Practices | Behaviors usually associated with a role that can be applied to a variety of goals in a variety of settings. For example, the practices of ‘budgeting’ are behaviors associated with the management role; ‘Procedure use’ is associated with the worker role; ‘Reinforcement’ is associated with leadership roles. |
| Precursor | According to Webster’s, that which occurs before, what exists prior to action. From the Latin praecurrere, to run before. See precursor event and error precursor, two distinct concepts. |
| Precursor Event | An event of low significance meeting three criteria: 1) No lasting state change in SSC or human/ organizational conditions occurs. 2) Nothing else occurs later as a result of the state change. 3) No reduction in the margin of safety occurred. Precursor events are also known as low-level events or near misses. |
| Process | A sequence of behaviors or series of steps designed to produce a product or service; tangible structures established to direct the behavior of individuals in a predictable, repeatable fashion as they perform various tasks. |
| Readiness | An individual’s mental, physical, and emotional preparedness to perform the job as planned. |
| Reinforcement | The consequences one encounters when a specific behavior occurs. |
| Resources | What is needed (means and circumstances) to perform the task as planned, such as equipment access, amenable ambient conditions, available tools, materials, protective clothing, and people. |
| Safety Culture | - “that assembly of characteristics and attitudes in organizations and individuals which establishes that, as an overriding priority, nuclear plant safety issues receive the attention warranted by their significance.” (IAEA, International Nuclear Safety Advisory Group, INSAG-4, “Safety Culture” 1994, p.7). - Shared beliefs, attitudes, and assumptions about hazards in the work place prevalent among station employees |
| Task | An activity with a distinct start and stop made up of a series of behaviors of one or more people. |
| Worker | An individual who takes action to change the plant state, or does maintenance, design, or construction of plant equipment (physical) or an individual who takes action to change plant controls, tools, design-bases documentation, procedures, administrative functions, support activities (paper). |